#### LOCAL BIODIVERSITY AREAS

A PILOT STUDY ON THE IDENTIFICATION AND EVALUATION OF LOCAL AREAS FOR WILDLIFE AND NATURE CONSERVATION

NOVEMBER 2005

Enterprise Centre, The Murrough, Wicklow

Tel: 0404-64020  Fax: 0404-64022

e-mail: info@natura.ie

#### LOCAL BIODIVERSITY AREAS

A PILOT STUDY ON THE IDENTIFICATION AND EVALUATION OF LOCAL AREAS FOR WILDLIFE AND NATURE CONSERVATION

NATURA Environmental Consultants

1.	INTRODUCTION	3

1.1.	The need for a lower tier of sites	3

1.2.	Background information	3

1.3.	Legislative and policy context	5

2.	REVIEW OF BEST PRACTICE ELSEWHERE	8

2.1.	Overview	8

2.2.	Case studies	13

3.	METHODOLOGY AND GUIDELINES FOR SITE SELECTION	15

3.1.	Introduction	15

3.2.	Methodology for identifying and surveying sites	15

3.3.	Setting criteria for site selection	18

4.	PILOT STUDIES	20

4.1.	County Wicklow Pilot Study	20

4.2.	County Westmeath Pilot Study	28

5.	CONCLUSIONS AND RECOMMENDATIONS	36

5.1.	Conclusions	36

5.2.	Recommendations:	39

6.	REFERENCES	41

7.	APPENDICES	44

Appendix 1:  System for the evaluation of sites for flora, fauna and fisheries in National Road Schemes.	44

Appendix 2:  Summary of consultations and information gathered	45

Appendix 3.  Draft Guidelines for the selection of biological, geological and geomorphological sites of local nature conservation importance in Northern Ireland.	46

Appendix 4.  Rarity value of habitat types listed in A Guide to Habitats in Ireland (Fossitt 2000)	54

Appendix 5.  Checklist of protected species in Ireland	57

Appendix 6:  Site synopsis for designated sites of conservation interest in Co. Wicklow	72

Appendix 7:  Site synopsis for designated sites of conservation interest in Co. Westmeath	77

Appendix 8.  The Irish Geological Heritage Programme	83

# INTRODUCTION

## The need for a lower tier of sites

In Ireland there are currently nineteen environmental designations for the protection of nature, the landscape and amenities, these were reviewed by Hickie (1997).

In terms of nature conservation designations Special Areas of Conservation (SACs) and Special Protection Areas (SPAs) give protection to plants, animals, birds, and wildlife habitats of international conservation significance under the EU Habitats Directive (Natura 2000 sites), whereas Natural Heritage Areas (NHAs) protect plants, animals, and wildlife habitats of national significance within Ireland.  We also have a number of Statutory Nature Reserves, National Parks, and Refuges for Fauna.  The hunting of wild birds is prohibited in Wildfowl Sanctuaries, and sites may also be designated as RAMSAR Sites, Biogenetic Reserves, or as a UNESCO Biosphere Reserve.

Some of our watercourses are also designated as Salmonid Waters under The Freshwater Fish Directive, and certain areas are proposed as Sensitive Areas for Urban Wastewater (under the EU Urban Wastewater Directive), or as Sensitive Areas for Fisheries and Forestry (proposed).  The EU Water Framework Directive will also protect River Basins.  All of the above designations are selected at national level and do not cover sites of less than national importance

Designations which protect amenity areas include Areas of Special Control in Development Plans, Special Amenity Area Orders, and Tree Preservation Orders but the conservation of wildlife or habitats is generally only a secondary effect of these designations.  These are the only form of designations currently in the direct control of local authorities which may have benefits for biodiversity conservation.

The absence of a reliable list of sites of local or regional value for biodiversity leaves a significant gap at the local level.  The result is that county development plans or local area plans frequently overlook local sites of value and place too much reliance on national or international designations.

## Background information

When the original list of Areas of Scientific Interest (ASIs) in Ireland was prepared by An Foras Forbartha (Anon 1981), these were classified into four categories of importance (International, National, Regional and Local).  Following the transfer of this system to the National Parks & Wildlife Service (now part of the Department of Environment, Heritage and Local Government), the ASIs were re-evaluated and the majority were proposed for designation as Natural Heritage Areas (pNHAs).  In the National ASI Survey Manual used by the NPWS, the following statement was included in Appendix VI.

Rating of sites

It is not proposed to continue the practice of rating sites except insofar as this is necessary to conform to internationally accepted methodology for selection of ornithological sites.  To some extent it invites planning authorities and development agencies to override the designation in the case of lesser rated sites and thus undermines the purpose of the entire exercise.  Sites of local importance are more appropriately conserved through measures which offer positive encouragement of good environmental practice, rather than negative constraints.  A re-evaluation of all ASIs is required since the ratings given to some are seen in retrospect to be incorrect.

Subsequent to the re-evaluation of ASIs, which began in the period 1993-94, the NHA network was established and these sites are generally treated as at least nationally important as they are now designated under national legislation (Wildlife (Amendment) Act, 2000).  However, the surveys of potential sites for proposal as NHAs have continued (for example with national inventories of raised bogs, blanket bogs, turloughs sand dunes, shingle beaches, lagoons, native woodlands and other habitat types).  In all cases, sites were recorded which would not qualify as NHAs, either due to small size or to the degree of modification of the habitat type.  Unfortunately, no system exists at present where such sites of local value may be listed and taken into account in the planning process.

The existence of a large number of locally important (undesignated) sites is also evident from the Environmental Impact Assessment procedures for large infrastructural developments such as roads.  The National Roads Authority has recently published guidelines for assessment of ecological impacts of National Road Schemes (Anon 2004).  These require the carrying out, for each scheme, of a route corridor constraints study to identify the main ecological constraints that should be avoided or that could affect the design of the scheme, delay progress or influence the costs.  This includes the listing of designated areas but also includes any other sites of ecological value, identified from aerial photographs, within or in close proximity to the study area.  A review of a sample of five such constraints studies shows that the majority of the sites identified are undesignated.  This varied from between 79% and 96% or all sites identified (Table 1).  The NRA guidelines also present a rating system for ranking sites of ecological value – these are presented in Appendix 1.

At present, such locally important sites are identified in a wide variety of environmental impact studies, many of which are carried out on behalf of local authorities.  In the absence of a system for recording the details, for listing and evaluating the sites for inclusion in local development planning, the information may be lost and no further account taken of the sites.

Table 1.  Number of sites of ecological value identified in road corridor constraints studies for five national road schemes (unpublished data from NATURA Environmental Consultants)

| National Road Scheme | N3 | N8 | N11 | N15 | N18 |
| --- | --- | --- | --- | --- | --- |
| County | Meath | Tipperary | Wexford | Donegal | Galway |
| Study area (km2 approx) | 132 | 166 | 80 | 42 | 135 |
| Total number of ecological sites identified | 23 | 120 | 26 | 11 | 68 |
| Percentage of undesignated sites in study area | 82% | 96% | 96% | 81% | 79% |

## Legislative and policy context

The following description of the EU Habitats, Bird, and Water Framework Directives and the Convention on Biological Diversity and the National Biodiversity Plan were provided within a submission by the Galway City Development Board’s Inventory of Nature and Wildlife Habitat sub-group to the Senior Executive planner in Galway City Council.  This comprehensive submission identified 11 sites of ‘Special Natural Heritage’ within the city and highlighted important ecological corridors within the city.

###### “EU Habitats and Bird Directives

All EU Member States have legal obligations to select and protect sites that contain habitats and/or species listed on annexes of the Habitats Directive and Birds Directive.  These are habitats and species that are considered to be under threat or in danger of disappearance, in the EU and for which the EU has particular responsibility.  Sites include Special Areas of Conservation (SACs) for habitats and species other than birds, and Special Protection Areas (SPAs) for birds.  SACs and SPAs are intended to form a coherent European network of protected sites known as the Natura 2000 network.  It should be noted that it is the Member State that is responsible for the protection of these sites, and that includes all local authorities.  Development Plans must include specific objectives for the conservation of European sites.

In addition to the protection of selected, qualifying sites (SACs and SPAs), Article 10 of the Habitats Directive also provides for the protection and management of areas outside, or inter-linking, these sites.

Article 10 (Habitats Directive)

‘Member States shall endeavour, where they consider it necessary, in their land-use planning and development policies and, in particular, with a view to improving the ecological coherence of the Natura 2000 network, to encourage the management of features of the landscape which are of major importance for wild fauna and flora.  Such features are those which, by virtue of their linear and continuous structure (such as rivers with their banks or the traditional systems for marking field boundaries) or their function as stepping stones (such as ponds or small woods), are essential for the migration, dispersal and genetic exchange of wild species.’

Article 10 thus encourages the management, maintenance and creation of ecological links, corridors or stepping-stones between SACs and SPAs through land-use planning and development policies.

This has been transposed into Irish law in Regulation 35(b) of the European Communities (Natural Habitats) Regulations, 1997, as follows:

“The Minister shall encourage the necessary research and scientific work for the purpose of meeting the requirements of Article 11 of the Habitats Directive with particular attention to scientific work necessary for the implementation of Articles 4 and 10 of that Directive.”

###### EU Water Framework Directive

The EU Water Framework Directive (WFD) establishes a comprehensive framework for the management, protection and enhancement of all inland and coastal waters, including groundwaters, and of all water-dependent habitats and species.  It replaces several existing directives that deal with specific threats to water quality and aquatic environments.  The WFD sets out to preserve, and where necessary, improve the quality of rivers, lakes, estuaries, coastal waters and groundwaters.  This represents a major challenge for the management of catchments and their waterbodies and wetlands in the future.  The WFD requires that reference conditions of good water quality must be established for all waters and this research work is ongoing at present.  Timeframes are also specified and good water quality must be achieved for all waters by 2015.

###### Convention on Biological Diversity and the National Biodiversity Plan

The UN Conference on Environment and Development held in Rio de Janeiro in 1992 resulted in the acceptance, among other things, of the Convention on Biological Diversity (CBD).  The CBD was signed by over 150 countries including Ireland, and by the EU.  Ireland ratified the CBD in 1996 and, in so doing, undertook to promote the conservation and sustainable use of biodiversity.  Two of the key objectives of the CBD are the conservation of biodiversity and the sustainable use of its components.

In order to ensure that Ireland fulfils its obligations under the CDB, the Department of Arts, Heritage, Gaeltacht and the Islands published the National Biodiversity Plan (NBP) in 2002 in order:

- To develop national strategies, plans or programmes for the conservation and sustainable use of biodiversity;

- To integrate, as far as possible and as appropriate, the conservation and sustainable use of biodiversity into relevant sectoral or cross sectoral plans, programmes and policies.

The NBP represents a new approach to conservation as the implementation of an overall coherent plan for biodiversity in Ireland has never before been attempted.  The NBP complements Ireland’s National Heritage Plan (2002), and takes cognizance of the European Community Biodiversity Strategy.  One of the key concepts is that individuals, communities, non-governmental organisations, regional and local authorities, Government Departments and agencies all share responsibility for the protection of biodiversity.  All local authorities are now obliged to prepare a Local Biodiversity Plan for their areas in consultation with the relevant stakeholders.

The NBP embraces the internationally recognized principles of sustainable development that should underlie environmental policy in Ireland:

- The precautionary principle;

- The integration of environmental considerations into all areas and across all sectors;

- The polluter-pays principle;

- The intergenerational principle;

- The principle of shared responsibility for environmental protection by public bodies, private enterprises and the general public.

Sustainable development presents a major challenge for all sectors in Ireland.  It calls for early insights into possible late consequences, and calls on all actors to contribute before the damage has happened.  It is a concept to enhance simultaneously the economic wealth, social cohesion, institutional effectiveness and environmental quality.  There can be no opponents to its acceptance as the only other alternative is unsustainability.  Providing knowledge and background information is not enough in itself – it must also be communicated and integrated into policy development”.

Local Biodiversity Plans

Action 10 of the National Biodiversity Plan states:

“Each Local Authority to prepare a Local Biodiversity Plan in consultation with the relevant stakeholders.

The initial focus of Local Biodiversity Plans will be on the identification and assessment of the local biodiversity resource.”

Draft Guidelines for the production of Local Biodiversity Action Plans (published by the Heritage Council 2003) state:

Establish a database:  The focus of the local database should be to generate and make accessible information on the biological diversity of the wider countryside, outside designated sites.

Audit of the local biodiversity resource:  This should identify features that are of:

- International and national interest, or

- locally distinctive, or locally rare and therefore of local conservation interest.

Local Authority Development Plans

Some Local Authority Development Plans include a reference to the need to identify and protect local biodiversity areas.  The following example from the County Wicklow Draft Development Plan 2004-2010, demonstrates the planning context:

Policy HL 12:

"The Council shall seek to identify, protect and conserve, in co-operation with the relevant statutory authorities and other groups a representative sample of the County's wildlife of local or regional importance, not otherwise protected by legislation."

Policy HL 13:

"The Council will seek to protect and enhance wherever possible wildlife habitats such as coast, cliff, dunes, rivers, streams, lakes, bog, fen, marsh, scrub and woodland, field boundaries (hedgerows, ditches and stone walls) which occur outside of designated areas.  Taken as a whole, such features form an important network of habitats and corridors which allow wildlife to exist and flourish."

# REVIEW OF BEST PRACTICE ELSEWHERE

At an early stage in this study, it was decided to confine the review of best international practice to the United Kingdom, to avoid difficulties in translation of documents and because a large amount of experience in this field has been accumulated in the UK.  Consultations with a wide variety of parties were carried out.  A list of all consultees is presented in Appendix 2.  In the UK the terms “Wildlife Site” or “Local Site” have been adopted as generic terms referring to all sites selected by systems that seek to identify non-statutory sites of importance within a local context.

## Overview

United Kingdom

An annual review of the status of UK Wildlife Site Systems is published by the Wildlife Trusts (e.g. Hackett 2003).  All local authority areas (131 administrative areas) in the UK have some form of Local Wildlife Site system.  Of these 81% of systems have a single partner taking lead responsibility for running the system, most commonly a Wildlife Trust or local authority.  The usual number of partners involved in UK Local Wildlife Site systems is four.  In total 103 systems have documented selection criteria and 57 systems have a formal panel of experts for selecting sites against these criteria.  In 104 system areas, local authority development plans have policies to protect Local Wildlife Sites.

England

The UK Department for Environment, Food and Rural Affairs has prepared a review of, and guidance on, the development and management of systems to identify sites of local importance for nature conservation in England (DEFRA 2005).  These are termed Local Sites systems which complement the internationally and nationally designated wildlife and geological sites.  Its main findings are:

- In most areas, local authorities, working with other local partners, have established systems of locally valued non-statutory sites.  These contribute significantly to delivering both UK and Local Biodiversity Action Plan targets.

- The lack of a consistent approach to the selection of sites and system management made it difficult to apply national or regional policies consistently or target national funding streams.

- Local sites fall within the Planning System which is a function of Local Authorities.

- The establishment and management of a Local Sites system should be based on a partnership approach, involving those organisations with an interest in biodiversity conservation.

- Local authorities should provide leadership in establishing and maintaining partnerships to identify and manage Local Sites.

- The voluntary and community sector will be key to the success of any Local Sites Partnership.

- The positive engagement and co-operation of landowners and their representative bodies can contribute significantly to the success of Local Sites partnerships.

- The key functions of a local sites partnership are to:

  - be responsible for site selection;

  - possess adequate technical knowledge;

  - develop and document site criteria;

  - select sites against criteria;

  - record reasons for selection of each site;

  - allow site owners an opportunity to comment;

  - de-select sites which no longer qualify.

- All local site systems should have a set of clear and locally defined site selection criteria with measurable thresholds.  The criteria thresholds for features of nature conservation interest will be set differently within one district or county compared to another.

- Where a locality has a number of small fragments of semi-natural habitat, which together could be managed to provide linkages for natural colonisation or movement, a Local Site boundary can be defined to take in a wider area and include both discontinuous patches and the areas separating them.

- The essential functions for which resources will need to be found for management of Local Site systems are:

  - Site records and information management;

  - Selection of Local Sites;

  - Advice and support for site management;

  - Community involvement and educational value;

  - Reviewing Local Sites systems and monitoring site conditions.

Scotland

A voluntary organisation, the Scottish Wildlife Trust (SWT) had previously published a manual for the survey, assessment and processing of Wildlife Sites (Halcrow, Somerville and Smart 1996).  The selection of Wildlife Sites in this survey was defined on the basis of the following criteria:

- Diversity of habitats and species;

- Rarity of habitats and species;

- Naturalness;

- Extent in a local context.

Each of these attributes was scored and when the sum of the scores exceeded a defined threshold the site was considered as a Wildlife Site.  Where a site was a borderline case, then other aspects relevant to wildlife conservation in a local context were reviewed.  These included vulnerability and access to the site for amenity or educational purposes.

Between 1997 and 2002, the SWT managed the Scottish Wildlife Action Programme which set out to identify, survey and assess sites across lowland Scotland.  The programme enabled 1,294 sites to be surveyed of which 514 sites acquired Wildlife Site status (Scottish Wildlife Trust 2002).

In Scotland, Local Sites are given recognition and protection through the planning system.  In 2001, 29 out of 32 local authorities in Scotland had developed some form of Local Site system with 23 systems making reference to ‘Wildlife Sites’.  In 2002, 13 out of 17 Structure Plans contained specific policies pertaining to the designation of Local Sites, with 10 of the plans making specific reference to wildlife site systems.

In June 2004, the Convention of Scottish Local Authorities and Scottish Natural Heritage (SNH) set out a new draft approach to Local Site designations in Scotland.  This promotes the name Local Biodiversity Sites (LBS) and proposes common terminology, conventions and minimum standards for sites across Scotland.

In 2005, SWT published its draft policy on Local Sites (Scottish Wildlife Trust 2005).  SWT believes that all Local Authorities in Scotland should take the responsibility for establishing and maintaining a list of Local Sites and protecting them through their development control policies.  SWT believes that one of the most effective ways of securing and enhancing the interest of a site is to engage and support the site owner.  It also believes that the interest on sites can be enhanced by integrating Local Site systems into agricultural subsidy programmes and land management incentive schemes.

Northern Ireland

The focus in Northern Ireland from 2002 onwards was on extensive survey to secure baseline data.  There is a recognition by the Environment and Heritage Service Northern Ireland (EHS) that robust guidelines for selecting sites are required.

This activity has coincided with a rolling programme of review of all area plans in Northern Ireland.  The area planning function is carried out by the Planning Service (part of the Department of Environment) and not by local authorities.  EHS contributes Local Site data to Planning Service for consideration in developing each draft plan.  A summary of the information that EHS supplied during the desktop review for this project is presented below.

“Legislative basis for designating areas of nature conservation importance in Northern Ireland.

The legislative basis for the designation of sites of national and international importance to nature conservation in Northern Ireland may be found in three separate pieces of legislation.  The Conservation (Natural Habitats Etc) Regulations (Northern Ireland) 1995 provides for such areas to be designated as Special Areas of Conservation and Special Protection Areas.  The Environment (Northern Ireland) Order 2002 legislates for Areas of Special Scientific Interest, whilst The Nature Conservation and Amenity Lands (Northern Ireland) Order 1985 (NCALO) legislates for National Nature Reserves and Marine Nature Reserves.

The NCALO also legislates for the designation of Local Nature Reserves by district councils, but to-date establishment has been slow with only six LNRs designated.  The fore-mentioned legislation provides no additional methods to designate and protect sites for their local importance to nature conservation.

Policy basis for designating local areas of nature conservation importance.

The Conservation (Natural Habitats, etc.) Regulations (NI) 1995, The Regional Development Strategy for Northern Ireland 2025 and The Northern Ireland Biodiversity Strategy 2000 require the retention, conservation, protection, enhancement, promotion and management of local biodiversity.  The Council of Europe Recommendation (Rec(2004)3) requires conservation of the geological heritage and areas of special geological or geomorphological interest.

Implementation of these environmental requirements is being assisted by the identification and designation of Sites of Local Nature Conservation Importance (SLNCIs) via the planning process during the preparation of Development Plans.

Sites of Local Nature Conservation Importance and Development Plans.

The Regional Development Strategy for Northern Ireland (RDS) provides an overarching framework for all development plans and planning policy.  Strategic Guideline ENV 1.2 of the RDS endorses the identification and designation of Sites of Local Nature Conservation Importance (SLNCIs) in development plans.

In addition Planning Policy Statements (PPSs) set out the policies of the Department of the Environment on particular aspects of land-use planning and apply to the whole of Northern Ireland.  PPS2, Planning and Nature Conservation, sets out the Department’s land-use planning aims and policies for the conservation of our natural heritage.  Paragraph 60 states that “in the course of drawing up a development plan, sites of potential nature conservation importance (including sites of earth science importance) may come to light.  These will be carefully evaluated and if confirmed to be of local nature conservation importance will be formally identified in a development plan.”  Development plans set out specific planning policies, which apply to development proposals within or adjacent to SLNCIs.

Representations in support of or objections to any of the SLNCIs may be received during public consultations of Draft Development Plans.  Objections may request for SLNCIs to be added or removed from the final plan.  EHS evaluates the worthiness of any suggested additional sites and defends the interests of any published site during subsequent inquiries that are held by the Planning Appeals Commission (PAC).  EHS then reviews the recommendations of the PAC and produces the list of SLNCIs that are to be adopted.

Draft SLNCI guidelines

A set of draft guidelines have been prepared (presented in full in Appendix 3)  which indicate the range of sites that can be proposed as Sites of Local Nature Conservation Importance and provide a basis for developing more comprehensive guidelines for the selection of SLNCIs for each Area Plan.

The guidelines are intended to categorise the biodiversity interest of sites in respect of four different SLNCI types which are selected because of their habitats, species, functional value or community value.  The habitat categories are divided into 14 broad types and the species categories into 4 broad types.  These are based primarily on the priorities contained in the Northern Ireland Biodiversity Strategy.  Functional values of sites are also considered in the selection process.  This means that sites can be included as wildlife corridors, wildlife stepping stones or where they are complementary to adjacent designated areas, to assist with the reversal of fragmentation.  Sites may also be included which are of particular value to local communities either as wildlife amenity sites, active conservation management sites or as sites with important local historical and cultural elements.

These guidelines have taken on board the views of the Environment and Heritage Natural Heritage Directorate, the main NGO conservation bodies that have responded to Development Plans, District Councils, the Planning Appeals Commission and the lessons learned from previous Development Plans.  The CNCC has also been consulted regarding their views on the draft guidelines.

Methodology

There are three strands to identifying Sites of Local Nature Conservation Importance in Northern Ireland.

- Experts from Environment and Heritage Service assess and propose Geological and geomorphological sites. Information such as that provided by the Northern Ireland Earth Science Conservation Review (ESCR) assists with this process.

- Experts from Environment and Heritage Service assess and propose habitat and species sites. Teams of consultants who are contracted by Environment and Heritage Service assist this process by undertaking detailed habitat surveys throughout Northern Ireland. Other sources of local information are used when available.

- Experts from Environment and Heritage Service assess and propose Functional and Community sites. Once the geological, geomorphological, habitat and species sites have been assessed a list of sites are sent to a team in Environment and Heritage Service who look for wildlife linkages between suggested sites or other sites of functional or community importance.

## Case studies

This review covers the criteria which are used in the selection of wildlife sites (of local importance for nature conservation) in three local authority areas in England, as follows:

  - Lancashire;

  - Derbyshire;

  - Leicester, Leicestershire and Rutland.

Each set of guidelines was reviewed for features which would be useful in an Irish context.

Common elements of county guidelines

- The guidelines are generally divided into two sections.  The first section covers the selection of sites on a habitat basis while the second provides species-based guidelines.

Habitat criteria

- Each of the main habitat groups (i.e. woodlands, grasslands, heaths and bogs, freshwater habitats) is first described in a national and local context.

- The main semi-natural habitat types are then listed with the quantified scoring systems for the presence of a minimum number of qualifying species of listed typical vascular plants, bryophytes or fungi.

- The habitat descriptions are normally based on a standard published classification system such as the National Vegetation Classification (NVC).

- Each habitat type is given a minimum threshold for inclusion of sites (some habitats may be included if there is a minimum of 0.5 hectares present while others may require a minimum area of 10 hectares).

Species criteria

Species criteria include the selection of:

- any site which supports a population of a plant or animal species which is protected under national legislation or which is considered endangered, vulnerable or sensitive in a county context (the latter category is based on county Red Data Books which are not available for Ireland).

- Any site which supports more than 0.5% of the national breeding or non-breeding population of any native bird species, or which supports a breeding population of a bird species which is rare in the county, or which supports a significant proportion of the county total of a population of a scarce breeding or non-breeding bird species.

- The criteria for other lesser known animal groups, such as amphibians, are based on population estimates which have been calculated for the UK as a whole and where the thresholds of what constitutes a significant local population.

# METHODOLOGY AND GUIDELINES FOR SITE SELECTION

## Introduction

International best practice is to use a set of selection criteria to evaluate local sites against a structured framework in order to demonstrate why some are selected as Local Biodiversity Areas and others are not.  Criteria for selecting sites should be based on a sound knowledge of the relative importance of habitats and species in an international, national and local context.  Where possible such criteria should be quantitative and should set thresholds for selection.  However, due to lack of baseline data, assessing a site against criteria will nearly always involve some element of professional judgement.

This chapter offers some guidelines on an approach to be followed.  Firstly, it deals with the practical methodology for identifying, data-gathering and surveying a range of sites.  Secondly, it reviews the possible development of selection criteria for Local Biodiversity Areas in Ireland.  It is not possible to be prescriptive as each local authority area will have different priorities and needs for the lower tier of sites.  It is based largely on the Wildlife Sites Handbook published by the Wildlife Trusts in the UK (Hawkswell 1997).

## Methodology for identifying and surveying sites

Definition of a study area/ creation of a provisional local biodiversity area site list:

Sites that may have potential for recognition as a local biodiversity area may be identified by using a variety of approaches.  If the local authority was to commission a study to identify local biodiversity areas this could be approached in several ways depending on the purpose of the study.  An area could be defined on a catchment basis (as in the Wicklow Pilot Study), a set area (as in the Westmeath Study), or as part of a local area plan.

Individual sites might also be identified and proposed by local landowners, naturalists, Wildlife Trusts, Naturalist’s Field Clubs, ecological consultants, National Parks and Wildlife Service, and conservation non-governmental organisations (NGOs), or through the production of local biodiversity action plans.

Ideally sites proposed for inclusion in the suite of local biodiversity areas should be clearly marked on a map with a reference name, a grid reference and a reason for their selection.  These sites can then be bundled together for field survey work.

Desktop review and consultation:

Once a study area has been defined or a suite of sites have been identified for survey a desktop review and consultation with the National Parks and Wildlife Service (Department of Environment, Heritage and Local Government), the EPA, and the Regional Fisheries Board should be carried out.  This is in order to identify:

- if there are any designated areas (or proposed designations) within the proposed site/study area (NHAs, SACs, or SPAs),

- if there are any rare or protected floral species within the proposed site/study area,

- if there are any rare or protected faunal species within the proposed site/study area,

- the fisheries value of any watercourses, lakes or reservoirs in the study area,

- the water quality of any inland surface waters.

Other sources of information on species and habitats within the proposed site/study are may be found by contacting:

- The local authority, which may have carried out an audit of all biological studies carried out within the county as part of the County Heritage Plan.

- Published literature, journals, etc.

- BirdWatch Ireland (BWI) for information on birds, Irish Wetland Bird Survey (I-WeBS) data, any known records of rare or protected species, breeding sites, etc.  Both BWI HQ and the local branch may have some information of relevance.  The BTO Breeding and Wintering Bird Atlases will also provide some information on the species of birds found within a 10 km square.

- Bat Conservation Ireland (BCI) for records of bat roosts or bat activity.

- Dragonfly Ireland for records of dragonflies.

- The Irish Peatland Conservation Council (IPCC) for information on peatlands, bogs, fens, etc.

- The Botanical Society of the British Isles Atlas (BSBI) for plant records found within a 10 km square.

- The Irish Whale and Dolphin Group (IWDG) for whale and dolphin sighting and stranding information.

- The Irish Tree Society for records of champion trees.

- Forestry information is available from the Forestry Service, Department of Agriculture and Food in digital format known as FIPS (Forest Inventory and Planning System).

- The National Biodiversity Network Gateway website (http://www.searchnbn.net/index_homepage/index.jsp) also contains information on some datasets that relate to Ireland and may be of some use.

Identification of sites for survey:

In addition to compiling a list of proposed sites the local authority should set out to identify sites of ecological interest during the planning process – this is especially feasible given the wide use of GIS and remote sensing techniques in local authorities.

Remote sensing (aerial photography and satellite imagery) and computer mapping techniques such as Geographical Information Systems (GIS) are now widely available for recording, analysing, storing and displaying large amounts of spatial and/or temporal data.  This data can be used for planning large scale studies such as a habitat map of a county, or smaller studies such as the identification of local biodiversity areas within a set study area.

A variety of different sets of aerial photographs is available.  There is full coverage of almost all of Ireland with vertical black and white aerial photographs at a scale of 1:40,000 from 1995 - these are available in hard copy.  During summer 2000 there was complete coverage of the country in colour – these aerial photographs are available both in hard copy and more recently as geo-referenced digital images which can be imported into a GIS programme.  These images are up-to-date and comparatively easy to interpret as they were taken in summer, a good time of the year for habitat recognition.  Local authorities typically have a set of these images available for use within their GIS department.  The Ordnance Survey has also re-flown the country in 2005 and these images will be available digitally soon.

These aerial photographs and satellite images can then be used to identify and categorise common habitats that cover large areas of the countryside (mainly habitats of urban areas or intensive farmland), and to identify areas of potential ecological value (local biodiversity areas) that require field survey.  Hard copy maps using the aerial photos can then be produced for field surveys which show the boundary of the proposed local biodiversity area and it‘s location in the landscape relative to other features.

Despite this potential, remote sensing techniques cannot distinguish many of the habitat groups or categories listed in Fossitt (2000) or to determine the quality of the habitat present (see Appendix 4) and in almost all cases the identification of local biodiversity areas will involve new survey work.  This means that there are currently no alternatives to site visits or ground surveys by trained surveyors, whatever the scale of the exercise.  As this is time-consuming and costly, the aim of any survey methodology should be to minimise the amount of time that has to be spent in the field by survey personnel.  This use of aerial photography and or satellite imagery greatly speeds up time in the field surveys.

Other sources of information for the identification of potential sites include the Ordnance Survey 6” series of maps (1:10,560) which often depict detailed information on the extent of notable habitats such as woodlands, marshes, and bog and are a valuable resource.

Field surveys:

Field surveys of identified sites should then be carried out by experienced ecologists.  A walkover survey of the proposed local biodiversity area should aim to describe and evaluate the habitats present (as per Fossitt (2000)), and to take note of any mammal activity in the immediate vicinity of the site (based on signs such as tracks, droppings, feeding signs, etc.) or potential roost sites for bats, nesting sites for birds of prey, breeding sites for amphibians, spawning grounds for fish, etc.

A record card or survey form should be created based on the fields within the local biodiversity database.  This serves two functions – a standardised approach to how information is recorded is used, and all data required for the survey is clearly shown and thus key information cannot be missed whilst in the field.

Reporting:

All sites visited should be clearly shown and their boundary digitised in a GIS package.  Sites should be given a unique code and site name.  All field card data should be entered into a database so that it can later be queried and relevant species or habitat data linked or exported to other biological datasets such as a County Biological Records Database.

## Setting criteria for site selection

Basis for the criteria

General guidelines for choosing and evaluating sites of nature conservation importance were first formulated by Ratcliffe (1977) in the UK Nature Conservation Review.  These are sometimes referred to as the ‘Ratcliffe Criteria’, representing general principals and factors to be taken into account when considering the nature conservation value of a given site, rather than defined or quantified factors to be assessed.  Collis and Tyldesley (1993) and the Wildlife Sites Handbook (Hawkswell 1997) give a modern interpretation and summary of these well-known guidelines with respect to Local Sites.

The Wildlife Sites Handbook advises that the criteria for the selection of Local Sites on habitat grounds should consider the following primary and secondary elements:

| Primary criteria | Secondary criteria |
| --- | --- |
| - Rarity | - Position in an ecological unit |
| - Size | - Potential value |
| - Naturalness/typicalness | - Fragility |
| - Diversity | - Educational/social value |

Structure of criteria

Site selection criteria are best structured with separate criteria for habitats and species.  However, in practice many sites will be selected due to the combination of both habitats and species which they support.  It will be necessary to have a system which is flexible enough to accommodate sites which qualify under a number of criteria.

Numerical scoring systems are often used in other countries to quantify the selection criteria.  However, these should be based on good baseline information, which is often lacking in Ireland for habitats and taxonomic groups of plants and animals.  The weighting in scoring systems is often difficult to justify objectively and in such cases should be avoided.

Habitat criteria

Habitat criteria are best structured according to the broad habitat groups (Level 1) of the Heritage Council scheme (Fossitt 2000) (See Appendix 4).  These include Freshwater, Grassland and Marsh, Heath and Dense Bracken, Peatlands, Woodland and Scrub, Exposed Rock/Disturbed Ground, Cultivated and Built land, and Coastland.  Within these categories, specific habitat types should be identified and criteria for inclusion defined.  Criteria should usually include an assessment of quality (such as the species richness or level of damage or modification of the habitat) and size (with a minimum area given for each habitat type).

Most areas of habitats of international or national importance should be included.  Only small fragments which are non-viable or of very poor quality should be excluded.  In some cases where these habitats are present (e.g. calcareous springs with tufa formation) they should be selected, irrespective of quality or size.

For locally rare or distinctive habitats, not considered to be either of international or national importance, only the more significant examples should be identified as LBAs.  Thresholds should be set so that the larger, higher quality sites are selected.

Species criteria

Species criteria should usually be structured according to taxonomic groups.  It is useful to refer to relevant lists identifying species of international, national and local importance (See Appendix 5).  Criteria should cover all taxonomic groups where thresholds can be set, even if the thresholds are known only for international or national importance.

Availability of data

The lack of data should not prohibit the development of criteria, in particular relating to national and international priority habitats and species.  Thresholds can be set for local priorities as further data become available.  Where a system has gaps in the criteria because of inadequate data then this should be clearly stated and filling these gaps should be a high priority.

# PILOT STUDIES

Two pilot studies were carried out as part of this project – one was located in County Wicklow and the other in County Westmeath.  Under Article 10 of the EU Habitats Directive wildlife corridors are considered to include:

‘Such features as those which, by virtue of their linear and continuous structure (such as rivers with their banks or the traditional systems for marking field boundaries) or their function as stepping stones such as ponds or small woods), are essential for the migration, dispersal and genetic exchange of wild species’.

Corridors can be described as regional (linking sites separated by many kilometres) or as local (linking habitat patches within sites) (Good, 1998).  The pilot studies for the project were therefore designed to identify local biodiversity areas using both approaches – as habitats along linear landscape features and as stepping stones between designated sites.

## County Wicklow Pilot Study

In County Wicklow the catchment area of Newtownmountkennedy River (which was chosen as a linear landscape feature) was used to define the study area (see Fig. 2).  The river corridor links two designated areas (Fig. 1).  This small river rises within Carriggower Bog (Site Code: 000716) in the townland of Ballinasoostia, on the Calary Plateau at the eastern edge of the Wicklow Mountains.  This area of transition mire is designated as both a Special Area of Conservation (SAC) (under the EU Habitats Directive), and a proposed Natural Heritage Area (pNHA) under the Wildlife Act.  The site synopsis for this site is presented in Appendix 6.

The Newtownmountkennedy River then flows east to the coast passing through the lands of Altidore Estate, Newtownmountkennedy Village, and Woodstock Estate before flowing through the low lying Kilcoole Marshes (Site Code: 004085) to enter the Irish Sea at a point known as The Breaches, on Leamore Strand.  This area is designated as a Special Protection Area for Birds (SPA) (under the EU Birds Directive).  This area of low lying marshes is part of a larger coastal wetland complex known as The Murrough Wetlands (Site Code: 002249 and 000730), which stretches from north of Wicklow Town 15 km north to Ballygannon.  This area supports a number of habitats listed on Annex I and species listed on Annex II of the EU Habitats Directive, and is thus designated as a pNHA and a SAC.  The site synopsis for this site is presented in Appendix 6.

Fourteen areas of potential ecological interest (ecological sites) were identified from aerial photography of the study area (Figure 3).  Their locations are shown on Figure 4 and numbered sites are shown on Figure 5.  These areas were then visited (where feasible) or viewed using binoculars from a distance.  The key habitats present plus signs of (or their potential to support) any mammals, amphibians, birds, or reptiles were noted (see Table 2).  Figures 6, 7 and 8 show the ecological sites of local biodiversity interest at a higher resolution (1:12,000).  At this scale other features in the landscape which link ecological sites become more apparent.  For example, a treeline of mature beech and oak trees links sites 2a (Altidore) and site 4 (Glendarragh).  This treeline also links to a mature hedgerow at its eastern end which links to site 2b (Newtownmountkennedy forestry).

Information on fisheries value was sourced from the Eastern Regional Fisheries Board and is included in Table 2.  Information on wildlife and protected species was sourced from NPWS and BirdWatch Ireland and is also presented in Table 2.  There are four records of rare and protected flora from within the study area but many of them have not been seen recently – these include cornflower (Centaurea cyanus), bird cherry (Prunus padus), the Killarney fern (Trichomanes speciosum), and subterranean clover (Trifolium subterraneum).  A review of the Native Woodland Survey commissioned by National Parks and Wildlife Service in 2003 - 2005 was also undertaken to determine if any woodland sites of ecological interest had been identified.

Data on water quality for the Newtownmountkennedy stream was sourced from the EPA website (epa.ie).  Water quality Q values based on biological monitoring of macroinvertebrates show that above the village of Newtownmountkennedy the stream is of fair quality and identified as unpolluted with a Q rating of 4-5.  Further downstream at other EPA water monitoring points, the Q value decreases to 3-4 and 3 indicating a decrease in water quality with slightly to moderately polluted status.

Table 2.  Local biodiversity areas identified within the Wicklow Pilot Study

| Site No. | Townland/Site name (if any) | Habitats present (as per Fossitt 2000) | Faunal species present or likely | Possible threats |
| --- | --- | --- | --- | --- |
| 1 | Carriggower Bog pNHA/SAC (Site Code: 000716) | PF3 Transition mire and quaking bog PF Fen WS1 Scrub | Rare transition mire invertebrates present, Snipe, Jack snipe and other wetland birds | Drainage |
| 2 | Newtownmountkennedy Stream (Overview. See 2a-2f for ecological sub-sites along the river) | Includes (See 2a- 2 f for sub sites): FW1 Eroding/ upland rivers FW2 Depositing lowland river WN5 Riparian woodland WN6 Wet willow-alder-ash woodland WD1 Broadleaved woodland WD2 Mixed broadleaved woodland/conifer plantation WD4 Conifer plantation WS1 Scrub GA1 Improved agricultural grassland GA2 Amenity grassland | Mammals include: Otter, mink, Numerous bat species (See 2b) Birds include: Kingfisher, dipper, Fisheries Value ( ERFB) Brown trout Sea trout at lower reaches | Felling of native species, pollution from agriculture, Amenity use and residential development |
| 2a | Altidore Estate and Demesne Native Woodland Survey Site No. 0793 | WN6 Wet willow-alder-ash woodland WN1 Oak-birch-holly woodland WD2 Mixed broadleaved woodland/conifer plantation WD4 Conifer plantation WD5 Scattered trees and parkland | Badger, hedgehog, fox, red squirrel, bats, deer Buzzard, sparrowhawk, woodcock, woodland birds Pheasant rearing and stocking | Amenity use, development, felling |
| 2b | Newtownmountkennedy Demesne and woodlands (Woods are Coillte owned and in Neighbour-wood scheme) Native Woodland Survey Site No. 0826 | WN1-Oak-birch –holly woodland WD2 Mixed broadleaved woodland/conifer plantation WD4 Conifer plantation WS2 Immature woodland WD5 Scattered trees and parkland | Woodland birds, potential nesting site for birds of prey Dipper nesting in bridge Badger 6 species of Bat (recorded by NPWS): Leisler’s, common and soprano pipistrelle, brown long-eared, Daubenton’s and Natterer’s | Amenity use, felling |
| 2c | Springmount | WD1 Broadleaved woodland WD2 Mixed broadleaved woodland/conifer plantation WS1 Scrub | Woodland birds and other native mammals. | Amenity use, residential development, vegetation clearance/ burning and tree felling |
| 2d | Kilmullin Native Woodland Survey Site No. 0990 | WD1 Mixed Broadleaved woodland WS1 Scrub WS2 Immature woodland WL2 Treeline (along river bank) GS4 wet grassland ED5 Refuse and other waste (Landfill site) | Woodland birds and other native mammals. | vegetation clearance, development pressure, expansion of landfill. |
| 2e | Woodstock/ Druids Glen | WD2 Mixed broadleaved woodland/conifer plantation GA2 Amenity grassland WD1 Broadleaved woodland | Kingfisher | Amenity use Alteration of river profile (current series of pools are impassable to trout) |
| 2f | Ballyloughlin | WL2 Treeline (along river bank) GA1 Improved agricultural grassland BC1 Arable land | Hedgerow and woodland bird species | Development pressure, agricultural run-off. |
| 3 | Ballinastoostia | WD3 Mixed conifer plantation | Woodland birds and possibly birds of prey | Felling |
| 4 | Glendarragh Native Woodland Survey Site No. 0793 | WD1 Broadleaved woodland WD2 Mixed broadleaved and conifer woodland WN6 Wet willow-alder-ash woodland | Woodland birds and other native mammals. | Cattle grazing and felling of native species |
| 5 | Kilmurry | WD4 Conifer plantation WD2 Mixed broadleaved and conifer woodland | Badger, fox, deer Woodland birds | Felling |
| 6 | Merepark/ Monalin | WL2 Treeline WS1 Scrub GA1 Improved agricultural grassland GS2 Dry meadow and grassy verge GS4 Wet grassland WL1 Hedgerows | Woodland birds and other native mammals .including deer | Housing developments |
| 7a +b | Ballygarret | WS1 Scrub WS2 Immature woodland | Hedgerow birds | Vegetation clearance / burning |
| 8 | Kilquade | WN6 Wet willow-alder-ash woodland FW2 Lowland depositing river WD1 Mixed broadleaved woodland | Woodland birds and other native mammals. | Development pressure, tree felling |
| 9 | Leabeg Upper | WN6 Wet willow-alder-ash woodland WS1 Scrub GS4 Wet grassland | Hedgerow birds Wintering waterfowl moving on to agricultural land from the breeches (see site 10) | Vegetation clearance |
| 10 | The Breaches The Murrough Wetlands pNHA/SAC (Site Code: 002249/000730) Kilcoole Marshes SPA (004085) | GS4 Wet grassland FS1 Reed and tall herb swamp LS1 Shingle beach CM Salt marsh CW1 Lagoon and saline lake | Wintering Waterfowl: Bewick’s swan, Whooper swan, Greenland white fronted geese, greylag geese, golden plover, kingfisher, sandwich tern, little tern, little egret | Land drainage, amenity, development pressure, coastal erosion |
| 11 | Easthill | WD2 Mixed broadleaved and conifer woodland WD1 Broadleaved woodland | Woodland birds and other native mammals. | Tree Felling |
| 12 | Ballyhorsey | WD1 Broadleaved woodland WS1 Scrub WD4 Conifer Plantation | Woodland birds and other native mammals. |  |
| 13 | Altidore (North) | WD1 Broadleaved woodland WD4 Conifer Plantation | Woodland birds and other native mammals. | Tree felling |
| 14 | Kilpedder South | WD1 Broadleaved woodland WD5 Scattered trees and parkland GA1 Improved agricultural grassland | Woodland birds and other native mammals (in particular Bat species). | Development pressure, tree felling. |

## County Westmeath Pilot Study

An area of land between two designated sites - Lough Derravaragh and Lough Owel in Co. Westmeath was selected for the purposes of this pilot study (See Figure 1.).  A study area of approximately 7.5 km by 4 km between the lakes was defined and is shown in Figure 2.  There are several existing sites designated within this study area.  These include Lough Derravaragh (Site Code: 00684), which is designated as a SPA, while Lough Owel (Site Code: 000688) is both an SAC and a SPA.  Within the study area there are two additional designated sites – one is Ballynafid Lake and Fen (Site Code: 000673) which is a pNHA, and Scragh Bog (Site Code: 000692) which is an SAC.  The site synopses for these sites are presented in Appendix 7 and their locations are illustrated in Figures 1 and 2.

Once the study area was defined consultations with National Parks and Wildlife Service, the Shannon Regional Fisheries Board, BirdWatch Ireland, Bat Conservation Ireland, the Botanical Society of the British Isles county recorder, and Coillte took place.

The rare and protected green-winged orchid (Orchis morio) was previously recorded from the shores of Lough Owel; while the round leaved wintergreen (Pyrola rotundifolia) is known from Ballynafid.

Data on water quality for the River Gaine was sourced from the EPA website (epa.ie).  Water quality Q values based on biological monitoring of macroinvertebrates show that the stream is identified as slightly polluted with a typical Q rating of 3-4.

A review of the aerial photography of the study area was then carried out (Figure 3).  Thirty areas of potential ecological interest (ecological sites) were identified within the study area.  Their locations are shown on Figure 4.  These areas were then visited (where feasible) or viewed using binoculars from a distance.  The key habitats present plus signs of (or their potential to support) any mammals, amphibians, birds, or reptiles were noted - see Table 2.

A review of the Westmeath Peatland Study commissioned by Westmeath County Council in 2004 and the Native Woodland Survey commissioned by National Parks and Wildlife Service in 2003 - 2005 was also undertaken to determine if any peatland/woodland sites of ecological interest had been identified.  The locations of the peatland sites are also shown on Figure 5.

Each local biodiversity area was then clearly referenced with a code/number and is illustrated at a scale of 1:10,000 on Figures 6, 7, 8, and 9.

The River Gaine (site 30) connects many of the sites identified and provides a suitable corridor for a variety of species to move along.

Table 3.  Local biodiversity areas identified within the Westmeath Pilot Study.

| Site No. | Townland/Site name (if any) | Habitats present (as per Fossitt 2000) | Faunal species present or likely | Possible threats |
| --- | --- | --- | --- | --- |
| 1 | Lough Owel (SAC/SPA) Site Code: 000688 | FL3 Limestone/marl Lake FS1 Reed and large sedge swamp | Mallard, shoveller, pochard, tufted duck, Greenland white-fronted goose, brown trout, Scharff’s charr. | Water abstraction for Mullingar, over fishing, eutrophication, amenity use. |
| 2 | Clanhugh Demesne Wood Clanhugh Demesne | WD1 Broadleaved woodland WD2 Mixed broadleaved woodland/conifer plantation WD4 Conifer plantation | Goldcrest, coal tit, long tailed tit, chaffinch, blackbird, robin, blue tit, pheasant. Potential nesting site for bird of prey. | Felling of native species. |
| 3 | Ballynafid | GS4 Wet grassland | May be used for feeding snipe and other waders. | Currently grazed by horses. |
| 4 | Ballynafid Lake and Fen (NHA) Site Code: 000673 Ballynafid Native Woodland Survey Site No. 1079 | FL4 Mesotrophic Lake FS1 Reed and large sedge swamp GM1 Marsh PF Calcareous fen WN6 Wet willow-alder-ash woodland WD1 Broadleaved woodland WS5 Recently felled woodland FW2 Lowland depositing river | A variety of coarse fish species including bream, tench, roach, rudd, carp, pike. Snipe, mallard, mute swan, kingfisher likely. This site holds important records for a variety of dragonfly and water beetles. | Angling, amenity use, drainage, non-native species in woodland. |
| 5 | Culleendarragh Wood Culleendarragh | WD1 Broadleaved woodland WS2 Immature woodland | Hedgehog, woodland birds. | Felling. |
| 6 | Ballynafid Wood Ballynafid | WD1 Broadleaved woodland WD4 Conifer plantation | Woodland birds, potential nesting site for birds of prey. Badger. | Amenity use, felling. |
| 7 | Scragh Bog SAC/NHA Site Code: 000692 Native Woodland Survey Site No. 1076 | PF3 Transition mire and quaking bog WN6 Wet willow-alder-ash woodland FW2 Lowland depositing river | Rare transition mire invertebrates present. Snipe and other wetland birds. | Drainage. |
| 8 | Garrysallagh/Down Wood Garrysallagh/Down Westmeath peatland site No. 68 | WD4 Conifer plantation FW2 Lowland depositing river | Woodland birds, potential nesting site for birds of prey. | Felling. |
| 9 | Garrysallagh/Down fen complex Garrysallagh/Down Westmeath peatland site No. 68 | PF Fen GS4 Wet grassland WS1 Scrub WN6 Wet willow-alder-ash woodland FW2 Lowland depositing river | Likely to support a variety of waders. | Drainage, water pollution. |
| 10 | Knightswood | WS1 Scrub GA1 Improved grassland | May support several mammals such as badger, rabbit, rat, etc. Variety of common countryside birds. | Vegetation clearance, burning. |
| 11 | Knightswood | GS4 Wet grassland | May provide feeding for a variety of waders. | Drainage. |
| 12 | Down | GS4 Wet grassland WS1 Scrub WS2 Immature woodland FW2 Lowland depositing river | Woodland birds, feeding waders. | Drainage. |
| 13 | Galmoylestown Lower Wood Galmoylestown lower | WN2 Oak-ash-hazel woodland | Woodland birds, badgers, potential roost and feeding areas for bats. | Felling, grazing. |
| 14 | Tyfarnham | WS1 Scrub | Woodland birds. | Felling, grazing. |
| 15 | Culleenabohoge fen complex Culleenabohoge Westmeath peatland site No. 60 | PF Fen PB1 Raised bog WD4 Conifer plantation GS4 Wet grassland WS1 Scrub FW2 Lowland depositing river | Woodland birds, badgers and other mammals in scrub, possible site for feeding waders. | Drainage, afforestation, felling, vegetation clearance. |
| 16 | Rock of Tyfarnham Westmeath peatland site No. 60 | WD4 Conifer plantation GS4 Wet grassland WS1 Scrub FW2 Lowland depositing river | Woodland birds, badgers and other mammals in scrub, possible site for feeding waders. | Drainage, felling, vegetation clearance. |
| 17 | Cruckboeltane | WS1 Scrub WS2 Immature woodland WL2 Treelines | Woodland birds, badgers and other mammals in scrub. Potential bat roosts in treeline of beech. | Felling. |
| 18 | Blackmiles Wood Blackmiles | WS2 Immature woodland | Woodland birds and other native mammals. | Felling. |
| 19 | Stonehall fen complex Stonehall Westmeath peatland site No. 62 | GS4 Wet grassland WS2 Immature woodland PB Fen | Feeding and breeding site for waders and other woodland birds. | Drainage, felling. |
| 20 | Lough Patrick Westmeath peatland site No. 61 | FL4 Mesotrophic lake FS1 Reed and large sedge swamp PF Fen GM1 Marsh GA1 Improved grassland FW4 Drainage ditches WL1 Hedgerows WS1 Scrub WS2 Immature woodland | Potential breeding and feeding site for waterfowl, amphibians, and bats. Important coarse fishing site, stocks of tench and carp. | Eutrophication, drainage, grazing. |
| 21 | Multyfarnham | GS4 Wet grassland WS2 Immature woodland FW2 Lowland depositing river | Feeding and breeding site for waders and other woodland birds. | Drainage, felling. |
| 22 | Lismalady fen complex Lismalady | WS1 Scrub PF Fen GS1 Dry calcareous and neutral grassland FL8 Artificial lakes and ponds | Feeding and breeding site for waders and other common birds, possible badger sett location, potential for amphibians, reptiles and aquatic invertebrates. | Dumping, quarry expansion, vegetation clearance. |
| 23 | Froghanstown Wood Froghanstown | WD4 Conifer plantation FW2 Lowland depositing river | Woodland birds and other mammals. | Felling. |
| 24 | Donore bog complex Donore Westmeath peatland site No. 45 | PB1 Raised bog GS Wet grassland FW Drainage ditches WD4 Conifer plantation | Variety of woodland birds, deer and other mammals. | Drainage, increased afforestation, peat cutting. |
| 25 | Tober woodland complex Tober | WD4 Conifer plantation WD1 Broadleaved woodland GS4 Wet grassland | Variety of woodland birds, deer and other mammals, potential bat roosts. | Felling, overgrazing. |
| 26 | Ballynakill | WL2 Treelines WD5 Scattered trees and parkland | Variety of woodland birds and other mammals, potential bat roosts. | Felling. |
| 27 | Castlewatty Donore wood Westmeath peatland site No. 39 | WL2 Treelines WD1 Broadleaved woodland WS2 Immature woodland WN6 Wet willow-alder-ash woodland PF2 Poor fen and flush GA1 Improved agricultural grassland FW2 Lowland depositing river | Otter, variety of woodland birds, waders, and other mammals. | Drainage, reclamation, planting of non-native species, grazing, other developments such as expansion of sewage treatment works, amenity use. |
| 28 | Donore | WN6 Wet willow-alder-ash woodland WD1 Broadleaved woodland WS2 Immature woodland | Otter, variety of woodland birds, potential bat roosts, badgers present. | Amenity use, littering. |
| 29 | Lough Derravaragh NHA/SPA Site Code: 000684 | FL3 Limestone/Marl lake PF2 Poor fen and flush FS1 Reed and sedge swamp WN6 Wet willow-alder-ash woodland PB1 Raised bog | Otter, variety of wildfowl including coot, pochard, tufted duck, and mute swan, and a variety of important insect species. Important mixed fishery with stocks of brown trout, pike, roach, tench, and bream. | Pollution, siltation, water abstraction, amenity use, eutrophication. |
| 30 | River Gaine | FW2 Lowland depositing river | Otter, kingfisher, fish, amphibians, and a variety of bird species. Important trout spawning and nursery river – feeder stream for Lough Derravaragh. | Pollution, siltation, water abstraction, eutrophication. |

# CONCLUSIONS AND RECOMMENDATIONS

## Conclusions

The need for a lower tier of sites

Clearly a lower tier of sites (termed ‘Local Biodiversity Areas’) is urgently required in the Republic of Ireland to prevent the continual loss of this locally important resource.

Legislation

The strongest legislative provision for Local Biodiversity Areas is the EU Habitat’s Directive (Article 10) which requires member states to protect through planning and development policies those features of the landscape which provide linear features or stepping stones for wildlife.

Such policies may be stated in local authority development plans or more specifically in Local Biodiversity Plans, the preparation of which is a targeted action through the planning and development control policies.

Links between designated areas

Many of the designated areas (SACs, SPAs, and pNHAs) are small, fragmentary remnants of formerly more widespread semi-natural habitats.  They now constitute islands of biodiversity in a sea of relatively impoverished habitats.  By establishing a network of similar, although less ecologically valuable, sites in the same area, a number of the animal species that need a complex of sites will be allowed to remain.  This is most relevant to birds and mammals but mobile invertebrates such as butterflies, dragonflies, some beetles and aquatic animals may also benefit.

For example, the hen harrier, a bird of prey species listed in Annex I of the EU Birds Directive, prefers to nest in ungrazed heather moorland although some have moved to young forestry plantations which provide similar structural habitats.  Their hunting ranges usually encompass a wider area including marginal farmland, peatland or other open habitats.  Thus a network of lower diversity sites near and between the main habitats can make the area more viable as a home range for such species.

The last of the undamaged raised bogs will obviously be in the higher tier of the network but cutover bogs can still hold a moderately high biodiversity and can act as secondary habitats for certain species.  Cutaway bogs with birch woods are a particularly valuable habitat for land bugs (Hemiptera), which use all parts of the tree at different stages of their life cycles (Feehan & O’Donovan 1996).  Peatlands generally hold few bird species at low density but the surrounding habitats, such as cutaway colonised by birch or wet grassland with rushes, often hold a more diverse community.  For example, snipe may nest on the bog but feed on worms and other invertebrates which they find by probing the soft soils in surrounding wet grassland (Feehan & O’Donovan 1996).  Westmeath County Council have recently commissioned an audit of the peatland resources of County Westmeath to be conducted (NATURA (2005)) and this study has identified several areas of fen and intact bog many of which could be protected as local biodiversity areas.

With aquatic habitats such as rivers and lakes, the links between the richer sites are of vital importance for the larger more mobile wildlife species.  Aquatic mammals such as otter will readily move through a canal between rivers and lakes provided that there is sufficient cover and food on the corridor.  The otter is listed in Annex II of the EU Habitats Directive but, because of its wide-ranging habits, it is not adequately protected by the upper tier of designated areas (SACs and NHAs).  It requires a series of linking habitats or corridors between its main breeding and feeding areas (Hayden & Harrington 2000).  Such links can be provided by a lower tier of sites even though some of these may be artificial in origin.

Buffers around designated areas

The concept of buffer zones around National Parks is well established as a way of reducing the impacts of intensive land uses on the edges of semi-natural areas (see for example Killarney National Park, Management Plan (Anon 1990)).  However, this approach has not yet been applied in the case of other designated areas such as SACs, SPAs and NHAs.  It could be envisaged that an area of native broadleaved woodland, for example, would be encircled by an area of mixed woodland or native scrub of lesser importance for biodiversity but of significant value in dampening edge effects for woodland wildlife.  Woodland birds and mammals, for example, are more influenced by the structure of the woodland than by the species of trees in the canopy.  Wetland habitats which are, by definition, transitional between aquatic and terrestrial ecosystems especially need buffers to protect them from damaging effects on surrounding land such as polluting run-off, desiccation due to drainage or disturbance (in the case of birds and mammals).  Thus, the protection of wet grassland or marsh in the area surrounding a fen can be beneficial to the more diverse habitat.

Habitat creation and restoration

Used strategically, habitat creation can help to buffer and link the increasingly fragmented, isolated and diminishing habitat patches of high value that remain (Gilbert & Anderson 1998).  An Irish habitat creation scheme that is already achieving good results in this area is the Anne Valley project in Co. Waterford.  This has set out to create a series of locally valuable wetland habitats linked by the River Anne with the overall objective of improving the general diversity of habitats and the linkages between them.  Another Irish scheme where habitat creation is promoted is the latest phase (3) of the Rural Environment Protection Scheme (REPS).  Farmers participating in the scheme can adopt various options within the scheme’s prescriptions.  For example under REPS measure 4 retain wildlife habitats, in addition to protecting existing habitats farmers have the option of creating a new habitat, planting broadleaved trees or enhancing nature corridors.

Habitat restoration is widely accepted as a valuable contribution to biodiversity conservation.  For example, in the EU Habitats Directive (92/43/EEC), conservation is defined as a series of measures to maintain or restore the natural habitats and populations of species of flora and fauna.  Habitat restoration is already being practiced in Ireland on a number of peatland sites (see review in Foss & O’Connell 1996).  This approach is particularly valuable in the case of sites of lower ecological value which may not be designated due to former habitat damage.

Best practice elsewhere

Best practice in the United Kingdom is for all local authorities to establish, in conjunction with local partnerships, a system of ‘Local Wildlife Sites’, which is protected through planning and development control policies.  In Northern Ireland, the regional development strategy provides the framework for the identification and designation of ‘Sites of Local Nature Conservation Importance’ through development plans.

Selection of Sites

Guidelines for the selection of local sites in Britain are quite specific and generally rely on a set of quantitative criteria which define thresholds for inclusion or exclusion of such sites.  This is supported by extensive and accessible local databases, biological record centres and publications on the wildlife habitats and species in each local authority area.

In Northern Ireland, where such background data is more limited, the guidelines for selection of ‘Sites of Local Nature Conservation Importance’ is more general but is linked to the priorities contained in the Northern Ireland Biodiversity Strategy.

## Recommendations:

Cleary there is a need in Ireland for a lower tier of sites of local value for nature conservation (here termed Local Biodiversity Areas or LBAs) to protect those areas which are of less than national or international value.

The role of the local authority

Each local authority should prepare and maintain a register of LBAs within its jurisdiction and should seek to protect these sites through the planning system.  Consideration should be given to prioritising the identification of LBAs in areas of highest development pressure and in the preparation of local area plans.

Selection of Local Biodiversity Areas

LBAs should be selected through the use of standard guidelines which clearly set out the methodology to use and to rely on best professional judgement and consultation in terms of site selection.  The selection criteria should make use where possible of any national or local databases of habitats or species.  The selection criteria should not include social or cultural values such as ‘intrinsic appeal’ or ‘visual attractiveness’ as these are not normally relevant to biodiversity value and are difficult to measure objectively.  LBAs should not be ranked in value or importance as this can be difficult to justify objectively and may lead to overlooking of those sites which are ranked lowest.  As the coastal boundary of local authority jurisdiction is normally fixed at the high water mark, the selection of LBAs will normally exclude intertidal and marine areas.

Consideration should be given to the inclusion of geological and geomorphological sites within the process, as although these do not contribute directly to local biodiversity, a suite of sites of geological interest is currently being identified by the Geological Survey of Ireland (see Appendix 8).  Many of these will become designated as NHAs in collaboration with the National Parks and Wildlife Service, but other sites are being promoted in a lower tier as County Geological Sites, which have no statutory protection, but may be included within County Development Plans.  Many counties have now adopted County Geological Sites into their development plans, and are promoting their interest through Heritage Plans.

Creation of a database

Ideally a standard database for storing all data relating to Local Biodiversity Areas would be created and then distributed amongst local authorities through the Heritage Officers for use in local biodiversity areas studies/inventories.  This database should ideally be designed using RECORDER 2000 and have GIS capabilities so sites can be clearly digitised and displayed on interactive maps for planning purposes.  The database should where possible contain information on both the species and habitats present and be compatible with a local biological records database so all data is easily transferable and our understanding of the natural history of the county is improved.  The structure and design of this database might be supported by the Heritage Council.

Recognition/protection of Local Biodiversity Areas

The primary mechanism for protection of LBAs should be the Local Authority Development Plan or the Local Area Plan.  Integration of LBAs into agri-environmental support schemes such as REPS or afforestation schemes such as the Native Woodland Scheme are the best ways of funding their management for conservation.  As most of Irelands non designated land is under agricultural management, agri-environment schemes such as REPS have the potential to contribute to the identification and management of local areas of biodiversity.  Since its establishment in the late 1990’s the scheme has developed to include more nature conservation elements with habitat protection and creation now an important element.  However, there is still potential to improve the scheme through appropriate active management and enhancement of habitats on agricultural land.

# REFERENCES

Anon (1981).  Areas of Scientific Interest in Ireland.  An Foras Forbartha.  Dublin.

Anon (1990).  Killarney National Park.  Management Plan.  The Office of Public Works.

Anon. (2002).  Planning Policy Wales.  Welsh Assembly Council.  www.wales.gov.uk

Anon (2003).  Conservation and Protection of Natural Heritage and Biodiversity in Galway City – Development Plan Review.  Unpublished submission to senior executive planner, panning department, Galway City Council.

Anon. (2003).  Guidelines for the Production of Local Biodiversity Action Plans.  Unpublished report for The Heritage Council.

Anon.  (2004). Guidelines for Assessment of Ecological Impacts of National Road Schemes.  National Roads Authority.  Dublin.

Anon. (2004).  County Wicklow Draft Development Plan 2004 – 2010.  Wicklow County Council.

Crowe, O.  (2005). Ireland’s Wetlands and their Waterbirds.  BirdWatch Ireland.  Newcastle.

DEFRA (2005).  Local Site Systems:  Guidance on their Development and Management.  Department for Environment, Food and Rural Affairs.  (Third draft in prep).

Derbyshire Wildlife Trust (2003).  Guidelines for the Selection of Wildlife Sites.  Derbyshire Wildlife Sites Handbook Volume 2.  Derbyshire Wildlife Trust.

EHS (2005).  Draft Guidelines for the Selection of Biological, Geological and Geomorphological Sites of Local Nature Conservation Importance.  Draft in prep., Environment and Heritage Service of Northern Ireland.

Feehan, J. & O’Donovan, G. (1996).  The Bogs of Ireland.  The Environmental Institute.  University College Dublin.

Foss, P.J. & O’Connell, C.A. (1996).  Irish Peatland Conservation Plan 2000.  Irish Peatland Conservation Council.  Dublin.

Fossitt, J.A. (2000).  A Guide to Habitats in Ireland.  The Heritage Council.  Kilkenny.

Gilbert, O.L. & Anderson, P.  (1998). Habitat Creation and Repair.  Oxford University Press.  Oxford.

Good, J. (1998).  The Potential Role of Ecological Corridors for Habitat Conservation in Ireland: A Review.  Irish Wildlife Manuals, No. 2, Dúchas – the Heritage Service.

Gwent Wildlife Trust.  (2004).   Guidelines for the Selection of Wildlife Sites in South Wales.  Prepared by Gwent Wildlife Trust on behalf of The South Wales Wildlife Sites Partnership.

Hackett, R. (2003).  Status of UK Wildlife Site systems 2002.  The Wildlife Trusts.  Newark.

Halcrow, V., Somerville, V. and Smart, G. (1996).  Wildlife Sites Manual:  for the survey, assessment and processing of Wildlife Sites.  Scottish Wildlife Trust.  Edinburgh.

Hawkswell, S.  (1997). The Wildlife Sites Handbook (Version 2).  The Wildlife Trusts.  Lincoln.

Hickie, D. (1997).  Evaluations of Environmental Designations in Ireland.  2nd edition.  Heritage Council.  Kilkenny.

Lancashire County Planning Department (1998).  Lancashire County Heritage Sites Scheme.  Biological Sites – Guidelines for Selection.  Lancashire County Planning Department.

Lockhart, N. Madden, B. Wolfe-Murphy, S. Wymer, E., Wyse Jackson, M. (1993).  National ASI Survey – Guidelines for Ecologists.  Dúchas – the Heritage Service.

NATURA (2001).  ECONET Ireland.  Report submitted as part of a Preliminary Study of the Needs Associated with a National Ecological Network (2000-LS-4-4).  EPA Report.

NATURA (2005).  WESTMEATH PEATLANDS STUDY - Study to establish the location, nature and extent of peatlands in Co. Westmeath.  Unpublished Report to the Heritage Council.

Nelson, B. and Thompson, R. (2004).  Dragonfly Ireland.  Museums and Galleries of Northern Ireland.

Newton, S., Donaghy, A., Allen, D., Gibbons, D. (1999).  Birds of Conservation Concern in Ireland.  Irish Birds, Vol. 6, pages 333 – 344.

Scottish Natural Heritage (2000).  Local Nature Reserves in Scotland: A Guide to the Selection and Declaration.  Scottish Natural Heritage.

Scottish Wildlife Trust (2002).  Scottish Wildlife Action Programme.  The State and Status of Wildlife Sites in Scotland.  Unpublished report.

Scottish Wildlife Trust (2005).  Scottish Wildlife Trust Policy on Local Sites 2005.  Draft policy document.

The Wildlife Trusts (2001).  Status of UK Wildlife Site Systems.  The Wildlife Trusts.

The Wildlife Trusts (2002).  Status of UK Wildlife Site Systems.  The Wildlife Trusts.

The Wildlife Trusts (2002 – present).  Site Matters - The Biannual Newsletter about Wildlife Sites.  The Wildlife Trusts.

The Wildlife Trusts Leicestershire and Rutland.  (2001).   Guidelines for the Selection of Sites of Importance for Nature Conservation in Leicester, Leicestershire and Rutland.  The Wildlife Trusts Leicestershire and Rutland.

# APPENDICES

## Appendix 1:  System for the evaluation of sites for flora, fauna and fisheries in National Road Schemes.

| Rating | Qualifying Criteria |
| --- | --- |
| A | Internationally important Sites designated (or qualifying for designation) as SAC* or SPA* under the EU Habitats or Birds Directives. Undesignated sites containing good examples of Annex I priority habitats under the EU Habitats Directive. Major salmon river fisheries. Major salmonid (salmon, trout or char) lake fisheries. |
| B | Nationally important Sites or waters designated or proposed as an NHA* or statutory Nature Reserves. Undesignated sites containing good examples of Annex I habitats (under EU Habitats Directive). Undesignated sites containing significant numbers of resident or regularly occurring populations of Annex II species under the EU Habitats Directive or Annex I species under the EU Birds Directive or species protected under the Wildlife (Amendment) Act 2000. Major trout river fisheries. Water bodies with major amenity fishery value. Commercially important coarse fisheries. |
| C | High value, locally important Sites containing semi-natural habitat types with high biodiversity in a local context and a high degree of naturalness, or significant populations of locally rare species. Small water bodies with known salmonid populations or with good potential salmonid habitat. Sites containing any resident or regularly occurring populations of Annex II species under the EU Habitats Directive or Annex I species under the EU Birds Directive. Large water bodies with some coarse fisheries value. |
| D | Moderate value, locally important Sites containing some semi-natural habitat or locally important for wildlife. Small water bodies with some coarse fisheries value or some potential salmonid habitat. Any water body with unpolluted water (Q-value rating 4-5). |
| E | Low value, locally important Artificial or highly modified habitats with low species diversity and low wildlife value. Water bodies with no current fisheries value and no significant potential fisheries value. |

*SAC = Special Area of Conservation

SPA= Special Protection Area

NHA= Natural Heritage Area

## Appendix 2:  Summary of consultations and information gathered

* denotes response received from individual and information passed on to NATURA

March

UK: Alex Tait of East Sussex Co. Council (Association of Local Government Ecologists ALGE)

IRL: Dr Grace O’ Donovan (Independent ecological consultant) *

IRL: Dominic Berridge (Heritage officer, Waterford Co. Council) *

IRL: Congella McGuire (Heritage officer Clare Co. Council)*

IRL: Dr. Julie Fossitt (Regional Ecologist, NPWS)*

April

UK: Julian Branscombe (Gwent Wildlife, Wales)*

UK: Caitríona Carlin (English Nature)*

IRL: Catherine Keena (Teagasc)

IRL: Rob Goodbody *

UK: Rachel Hackett (Wildlife Trust UK)*

UK: Stewart Hay (Scottish Wildlife Trust) *

IRL: Aileen O’ Sullivan (Coillte)

May

UK: Dermot Hughes (Ulster Wildlife Trust)*

UK: Joyce Mc Cormick, Richard Weyl (Environment and Heritage Service Northern Ireland)*

IRL: Don Cotton (Sligo Institute of Technology)

IRL; Martin Kelly (Eastern Regional Fisheries Board)*

IRL: Gerry Clabby (Heritage Officer, Fingal Co. Council, Dublin)*

June

UK: Leicestershire Environmental Resources Centre*

UK: Len Wyatt (Environmental Science Advisor, Cymru Welsh Assembly Gov.)*

IRL: Meeting with Gerry Clabby and Hans Visser: Fingal Co. Council

November

IRL:  Sarah Gatley (Geological Survey of Ireland)*

UK:  Brian Nelson (Ulster Museum)*

IRL:  Tina Aughney (Bat Conservation Ireland)*

IRL:  Matt Hammond (Shannon Regional Fisheries Board)*

IRL:  Olivia Crowe (BirdWatch Ireland)*

IRL:  Dick Coombes (Bird Watch Ireland)*

IRL:  Dr. Stephen Newton (BirdWatch Ireland)*

IRL:  Brian Keeley (Bat Conservation Ireland)

IRL:  David Norris (National Parks and Wildlife Service)*

IRL:  Katharine Duff (NATURA Environmental Consultants)*

IRL:  Dr. Naomi Kingston (National Parks and Wildlife Service)*

UK: Mark Hammond (Environment and Heritage Service Northern Ireland)*

IRL: Dr. Jim Martin (Native Woodland Survey co-ordinator, BECC consultants)*

## Appendix 3.  Draft Guidelines for the selection of biological, geological and geomorphological sites of local nature conservation importance in Northern Ireland.

INTRODUCTION

The Regional Development Strategy for Northern Ireland (RDS) sets out a strategic planning framework to guide physical development within the region.  It provides an overarching framework for all development plans and planning policy.  Strategic Guideline ENV 1.2 of the RDS endorses the protection and management of areas designated for their scientific interest and supports the identification and designation of Sites of Local Nature Conservation Importance (SLNCIs) in development plans.

In addition Planning Policy Statements (PPSs) set out the policies of the Department of the Environment on particular aspects of land-use planning and apply to the whole of Northern Ireland.  PPS2, Planning and Nature Conservation, sets out the Department’s land-use planning aims and policies for the conservation of our natural heritage.  Paragraph 60 refers to development affecting Sites of Local Nature Conservation Importance as identified in development plans.  The paragraph states that in the course of drawing up a development plan, sites of potential nature conservation importance (including sites of earth science importance) may come to light.  These will be carefully evaluated and if confirmed to be of local nature conservation importance will be formally identified in a development plan.

Development plans set out specific planning policies, which apply to development proposals within or adjacent to SLNCIs.  These policies should make a significant contribution to the EU target of halting the loss of biodiversity by 2010 and should assist in the implementation of the Conservation (Natural Habitats, etc.).  Regulations (NI) 1995 by protecting landscape features that are essential for the migration, dispersal and genetic exchange of wild species.

The designation of SLNCIs and associated policies should also assist in the implementation of the Council of Europe Recommendation (Rec(2004)3) “on conservation of the geological heritage and areas of special geological interest” whereby member states through “sustainable development and restoration should respect and reflect the natural patterns and processes: the geology, the geomorphology and the soils.”

EHS Natural Heritage has the role to identify potential SLNCIs and to propose sites to be designated as SLNCIs through the development plan process.

BIOLOGICAL SITES OF LOCAL NATURE CONSERVATION IMPORTANCE

THE PURPOSE OF DESIGNATING BIOLOGICAL SLNCIs

Primary

- To contribute to halting the loss of biodiversity in Northern Ireland by identifying and protecting sites of substantive biodiversity value for local habitats or species

- To give protection to a network of sites for wildlife movement and for local communities and organisations to enjoy, conserve and study wildlife

Secondary

- To generate information on local sites that could be used to inform the targeting of local biodiversity action.

- To contribute towards the implementation of international, national and regional legislation, environmental strategies and policy.

- To conserve the diversity of habitats which enhance and contribute to the local landscape character and identity of Northern Ireland

GUIDELINES

The guidelines are intended to categorize the biodiversity interest of sites in respect to four different SLNCI types: habitats (HAB); species (SPP); function (FU) and community (CO).  These are detailed in Appendix 1.

In all cases sites are selected because they have substantive nature conservation value.

Habitats and Species Categories

These are primarily based on the priorities contained in the Northern Ireland Biodiversity Strategy and the network of statutory nature conservation sites.  The habitat categories (HAB) are divided into 14 broad types.  These are broader than priority habitats.  Likewise the species categories (SPP) are divided into four broad types.  These relate specifically to priority species, but also include categories for other species, species assemblages or populations which are of biodiversity significance.  It is intended that the presence of priority habitats and species within each SLNCI is noted in site descriptions where possible.  HAB and SPP categories can be found in Appendix 1.

During site selection, both local and regional rationale will be considered.

The regional considerations are used to identify criteria for the selection of sites throughout Northern Ireland.  These reflect the features used to select ASSIs i.e. habitat, extent/population size, diversity, naturalness, rarity and typicalness.

Local consideration is given to ensure the selection of sites is modified to take into account geographical distribution and local requirements.

Functional Categories

These are used to identify areas which are considered to be of substantive value in maintaining biodiversity at a local level by virtue of their habitat structure and geographical position

Wildlife Corridors (FU1) are important sites for the movement and dispersal of wildlife.  Wildlife corridors include linear features with a semi-natural structure such as rivers and banks, field boundary systems, coastal habitats, broadleaf or mixed broadleaf and conifer woodland, canals and dismantled railways.

Wildlife stepping stones (FU2) are discrete sites which support local biodiversity and can act a refuge for wildlife.  They may include habitats such as ponds, woods, lakes, peat bogs, coastal habitats, broadleaf woodland, scrub, fens and disused quarries.

The functioning of designated sites of international and national nature conservation importance is often enhanced by adjacent habitats of lower intrinsic interest.  Complementary designation sites (FU3) are sites that complement or enhance habitats associated with adjacent designated sites of international and national nature conservation interest or assist in the reversal of fragmentation.

Community sites

Local communities use wildlife sites to enjoy, conserve and study native biodiversity.  This includes utilising sites which are actively managed to conserve or enhance local bio-diversity.  Communities also value natural elements that form part of the history of the local area.

Wildlife amenity sites (CO1) are sites with wildlife interest that are used by the public for amenity purposes (e.g. environmental education, wildlife watching or passive recreation).

Conservation sites (CO2) are sites that are actively managed by organizations or individuals for habitat creation, regeneration or nature conservation.

Historical and cultural elements (CO3) are sites with natural features that have shaped local landscapes and influenced the culture and history of the local area.

GEOLOGICAL AND GEOMORPHOLOGICAL SITES OF LOCAL NATURE CONSERVATION IMPORTANCE

THE PURPOSE OF DESIGNATING GEOLOGICAL AND GEOMORPHOLOGICAL SLNCIs

- To support the development of a network of sites which collectively demonstrates the geological history of Northern Ireland, including past and present geomorphological processes.

GUIDELINES

The guidelines are intended to categorize the earth science interest of sites within three different earth science SLNCI categories: exposure sites, integrity sites and process sites – active and fossil.  These site categories cut across the range of geological and geomorphological themes relevant to Northern Ireland.  For practical purposes it is simplest to consider the SLNCI series under the three categories detailed below.

Exposure Sites

These sites are selected as they provide access to notable rocks and other geological strata that are otherwise buried and inaccessible.  Sites are typically quarries, coastal and river localities or mountain crags.  Key considerations for their conservation are retention of the exposure and the ability to access the site by foot.

Integrity Sites

These sites are selected as they contain rare or unusual geological features, of limited occurrence.  They can include notable fossil or mineral sites, and unusual rock types.  Sites are typically quarries although some coastal and river localities or mountain crags are also significant.  These features tend to be of very limited extent and so the key considerations for their conservation are retention of the entire feature of interest and the ability to access the site by foot.

Process Sites – Active And Fossil

These sites are important for their landforms and other features related to geological processes which are still operating (Active) or are historical (Fossil).  Active sites include coastal, river and cave systems.  Fossil sites include glacial landforms and features related to former higher sea-levels.  These features can be of considerable extent and often are very important determinants of landscape appearance and quality.  Some sites my have high potential economic value as aggregate deposits.  Key considerations for the conservation of these sites are retention of typical landforms and ensuring that active processes, where relevant, continue uninterrupted.

APPENDIX 1

Categories based on habitat

- HAB1: Native woodlands where there is evidence to suggest an ancient origin (includes upland oakwoods (UO), upland mixed ashwoods (MA), and wet woodland (WW)) supporting a range of indicator species.

- HAB2: Long-established areas of wood-pasture, parkland and demesnes (WP) supporting a range of indicator species.

- HAB3: Other woodlands (OW) or areas of scrub which add significantly to the woodland and biodiversity resource within the Plan Area.

- HAB4: Linear stretches of ancient and/or species-rich hedgerow (AH) or hedge banks which add significantly to the woodland and biodiversity resource within the Plan Area.

- HAB5: Areas of grazing marsh or low lying wet grassland supporting a range of typical species (e.g. breeding waders)(GM).

- HAB6: Areas of species-rich grassland supporting a range of indicator species (may include lowland meadows (LM), upland meadows (UM), lowland and upland calcareous grassland (LC/UC), limestone pavement (LP) and lowland dry acid grassland (LD).

- HAB7: Lowland heath (LH) or upland heath (UH) which supports a range of indicator species.

- HAB8: Wetland, including fen (FE), marsh, swamp and reed beds (RE) which support a range of typical species.

- HAB9: Lowland raised bog (RB) or blanket bog (BB), supporting a range of typical species.

- HAB10: Good quality lakes and ponds that support a range of typical species (may include mesotrophic lakes (MS) or eutrophic standing waters (EU)).

- HAB11: Rivers or streams (RS) known to support UK BAP species or rare or scarce species in Northern Ireland.  Good quality stretches of rivers that are likely to support these species.

- HAB12: Coastal habitats such as maritime cliff (MC), sand dunes (SD), and salt marsh (SM) which support a range of typical species.

- HAB13: Intertidal habitats, including mudflats (MF) and sea grass beds (SB) which support a range of typical species.

- HAB14: Sites with combination of two or more individual habitats that when considered as a mosaic, provide substantive nature conservation quality.

APPENDIX 1 (cont.)

Categories based on species

- SPP1: Site used by Northern Ireland priority species as listed in the Northern Ireland Biodiversity Strategy.

- SPP2: Site containing other species of biodiversity significance.

- SPP3: Site which supports species assemblages of biodiversity significance.

- SPP4: Site which is important for populations of species (e.g. birds).

Categories based on function

- FU 1: Wildlife corridors (linear features with a semi-natural structure).

- FU 2: Wildlife stepping stones (semi-natural habitats that act as refuges for wildlife or stepping stones).

- FU 3: Complementary designation sites (sites that complement or extend habitats associated with adjacent International or National designated sites of nature conservation interest).

Categories based on community value

- CO 1: Wildlife amenity sites (sites with wildlife and amenity interest).

- CO 2: Conservation sites (sites that are managed for nature conservation).

- CO 3: Historical and cultural elements (natural features that have shaped local landscapes and influenced the culture and history of the local area).

APPENDIX 2

Geological themes

Stratigraphic	Subject Block

Tertiary

Cretaceous

Jurassic

Triassic

Permian

Carboniferous

Devonian

Silurian

Ordovician

Dalradian

Moinian

Igneous		Subject Block

Tertiary

Carboniferous

Old Red Sandstone

Ordovician

Caledonian

Structural		Subject Block

Post Variscan

Variscan

Caledonian

Palaeontology	Subject Block

Tertiary

Cretaceous

Jurassic

Triassic

Permian

Carboniferous

Silurian

Ordovician

Mineralogical	Subject Block

Mineralogy And Metallogenesis

Pleistocene		Subject Block

Midlandian Deposits

Inter-Glacial Deposits

Pre-Midlandian

Upland glacial landforms

Holocene		Subject Block

Coastal Processes

Holocene Sea Level Changes

Mass Movement

Peat and Related Stratigraphy

Diatomite

Karst and Caves

## Appendix 4.  Rarity value of habitat types listed in A Guide to Habitats in Ireland (Fossitt 2000)

| CODE | HABITAT TYPE | RARITY VALUE AT NATIONAL OR INTERNATIONAL LEVEL |
| --- | --- | --- |
| F | FRESHWATER |  |
| FL FL1 FL2 FL3 FL4 FL5 FL6 FL7 FL8 | Lakes and ponds Dystrophic lakes Acid oligotrophic lakes Limestone/marl lakes Mesotrophic lakes Eutrophic lakes Turloughs Reservoirs Other artificial lakes and ponds | High High High High High High - Moderate Moderate |
| FW FW1 FW2 FW3 FW4 | Watercourses Eroding/upland rivers Depositing/lowland rivers Canals Drainage ditches | High High High Low |
| FP FP1 FP2 | Springs Calcareous springs Non-calcareous springs | High High |
| FS FS1 FS2 | Swamps Reed and large sedge swamps Tall-herb swamps | High High |
| G | GRASSLAND AND MARSH |  |
| GA GA1 GA2 | Improved grassland (modified) Improved agricultural grassland Amenity grassland (improved) | Low Low |
| GS GS1 GS2 GS3 GS4 | Semi-natural grassland Dry calcareous and neutral grassland Dry meadows and grassy verges Dry-humid acid grassland Wet grassland | High Moderate Moderate Moderate |
| GM GM1 | Freshwater marsh Marsh | High |
| H | HEATH AND DENSE BRACKEN |  |
| HH HH1 HH2 HH3 HH4 | Heath Dry siliceous heath Dry calcareous heath Wet heath Montane heath | High High High High |
| HD HD1 | Dense bracken Dense bracken | Low |
| P | PEATLANDS |  |
| PB PB1 PB2 PB3 PB4 PB5 | Bogs Raised bog Upland blanket bog Lowland blanket bog Cutover bog Eroding blanket bog | High High High |
| W | WOODLAND AND SCRUB |  |
| WN WN1 WN2 WN3 WN4 WN5 WN6 WN7 | Semi-natural woodland Oak-birch-holly woodland Oak-ash-hazel woodland Yew woodland Wet pedunculate oak-ash woodland Riparian woodland Wet willow-alder-ash woodland Bog woodland | High High High High High High High |
| WD WD1 WD2 WD3 WD4 WD5 | Highly modified/non-native woodland (Mixed) broadleaved woodland Mixed broadleaved/conifer woodland (Mixed) conifer woodland Conifer plantation Scattered trees and parkland | High Moderate Moderate Low Moderate |
| WS WS1 WS2 WS3 WS4 WS5 | Scrub/transitional woodland Scrub Immature woodland Ornamental/non-native shrub Short rotation coppice Recently-felled woodland | Moderate Moderate Low Moderate Moderate |
| WL WL1 WL2 | Linear woodland/scrub Hedgerows Treelines | Moderate Moderate |
| E | EXPOSED ROCK/DISTURBED GROUND |  |
| ER ER1 ER2 ER3 ER4 | Exposed rock Exposed siliceous rock Exposed calcareous rock Siliceous scree and loose rock Calcareous scree and loose rock | Moderate High |
| EU EU1 EU2 | Underground rock and caves Non-marine caves Artificial underground habitats |  |
| ED ED1 ED2 ED3 ED4 ED5 | Disturbed ground Exposed sand, gravel or till Spoil and bare ground Recolonising bare ground Active quarries and mines Refuse and other waste | Low Low Low Low Low |
| B | CULTIVATED AND BUILT LAND |  |
| BC BC1 BC2 BC3 BC4 | Cultivated land Arable crops Horticultural land Tilled land Flower beds and borders | Low Low Low Low |
| BL BL1 BL2 BL3 | Built land Stone walls and other stonework Earth banks Buildings and artificial surfaces | Low Low Low |
| C | COASTLAND |  |
| CS CS1 CS2 CS3 | Sea cliffs and islets Rocky sea cliffs Sea stacks and islets Sedimentary sea cliffs | High High High |
| CW CW1 CW2 | Brackish waters Lagoons and saline lakes Tidal rivers | High Moderate |
| CM CM1 CM2 | Salt marshes Lower salt marsh Upper salt marsh | High High |
| CB CB1 | Shingle and gravel banks Shingle and gravel banks | High |
| CD CD1 CD2 CD3 CD4 CD5 CD6 | Sand dune systems Embryonic dunes Marram dunes Fixed dunes Dune scrub and woodland Dune slacks Machair | High High High High High High |
| CC CC1 CC2 | Coastal constructions Sea walls, piers and jetties Fish cages and rafts | Low Low |

## Appendix 5.  Checklist of protected species in Ireland

KEY TO TABLE:

European Legislation

EU HD – EU Habitats Directive (Council Directive 92/43/EEC)

II – Annex II animal and plant species

IV – Annex IV animal and plant species

V – Annex V animal and plant species

EU BD – EU Birds Directive (Council Directive 79/409/EEC)

I – Annex I bird species

National legislation

WA – Wildlife Act (1976) & Wildlife (Amendment) Act (2000)

P – Protected species

FPO – Flora Protection Order

RDB – red Data Book Category

Ex- extinct

E – endangered

V – vulnerable

R – rare

I – indeterminate

II – internationally important

-  - species not listed

# - status unknown

Table 1.  Checklist of protected species in Ireland

|  |  | EU HD | EU BD | RDB | WA | Bern | Bonn | Preferred habitat type(s) | Geographic distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAMMALS |  |  |  |  |  |  |  |  |  |
| Whiskered Bat | Myotis mystacinus | IV | - | I | P | II | - | Summer roosts in buildings | Widespread |
| Natterer's Bat | Myotis nattereri | IV | - | I | P | II | - | Summer roosts in buildings | Widespread |
| Daubenton's Bat | Myotis daubentoni | IV | - | II | P | II | - | Bridges, buildings, caves, trees | Widespread |
| Leisler's Bat | Nyctalus leisleri | IV | - | II | P | II | - | Summer roosts in buildings | Widespread |
| Pipistrelle | Pipistrellus pipistrellus | IV | - | II | P | III | - | Summer roosts in buildings | Widespread |
| Brown Long-eared Bat | Plecotus auritus | IV | - | II | P | II | - | Summer roosts in buildings | Widespread |
| Lesser Horseshoe Bat | Rhinolophus hipposideros | II, IV | - | II | P | II | - | Summer roosts in buildings | West and SW only |
| Grey Seal | Halichoerus grypus | II, V | - | - | P | - | - | Rocky shores and cliffs | Coastal only |
| Common Seal | Phoca vitulina | II, V | - | - | P | - | - | Sheltered shores | Coastal only |
| Bottle-nosed Dolphin | Tursiops truncatus | II, IV | - | - | P | - | - | Open sea | Coastal only |
| Harbour Porpoise | Phocoena phocoena | II, IV | - | - | P | - | - | Open sea | Coastal only |
| Otter | Lutra lutra | II, IV | - | II | P | II | - | Watercourses, coast | Widespread |
| Badger | Meles meles | - | - | II | P | III | - | Woodland and many others | Widespread |
| Irish Hare | Lepus timidus hibernicus | V | - | II | P | III | - | Grassland, heathland, bog | Widespread |
| Hedgehog | Erinaceus europaeus | - | - | II | P | III | - | Woodland, scrub, hedgerow | Widespread |
| Pine Marten | Martes martes | V | - | II | P | III | - | Woodland, scrub | Mainly west and midlands |
| Pygmy Shrew | Sorex minutus | - | - | - | P | - | - | Diverse | Widespread |
| Red Squirrel | Sciurus vulgaris | - | - | - | P | - | - | Woodland | Widespread |
| Irish Stoat | Mustela erminea hibernica | - | - | - | P | - | - | Diverse | Widespread |
| Red Deer | Cervus elaphus | - | - | - | P | - | - | Woodland, upland | Widespread |
| Fallow Deer | Dama dama | - | - | - | P | - | - | Woodland | Widespread |
| Sika Deer | Cervus nippon | - | - | - | P | - | - | Woodland, upland | Widespread |
| Ship Rat | Rattus rattus | - | - | R | - | - | - | Buildings | Lambay Island only |
| BIRDS |  |  |  |  |  |  |  | Breeding habitat (except where stated) |  |
| Red-throated Diver | Gavia stellata | - | I | R | P | II | - | Breeds upland lakes | Donegal only |
| Black-necked Grebe | Podiceps nigricollis | - | - | R | P | II | - |  | Extinct |
| Storm Petrel | Hydrobates pelagicus | - | I | II | P | II | - | Offshore islands | Coastal only |
| Leach's Petrel | Oceanodroma leucorhoa | - | I | - | P | # | # | Offshore islands | Coastal only |
| Gadwall | Anas strepera | - | - | R | P | III | II | Lowland lakes | Localised |
| Pintail | Anas acuta | - | - | R | P | III | II | Lowland lakes | Localised |
| Garganey | Anas querquedula | - | - | R | P | III | II | Lowland lakes | Localised |
| Shoveler | Anas clypeata | - | - | R | P | III | II | Lowland lakes and rivers | Localised |
| Pochard | Aythya ferina | - | - | R | P | III | II | Lowland lakes and rivers | Localised |
| Common Scoter | Melanitta nigra | - | - | E | P | III | II | Lowland lakes | Mainly west |
| Goosander | Mergus merganser | - | - | R | P | III | II | Rivers | Localised |
| Bewick's Swan | Cygnus columbianus bewickii | - | I | - | P | # | # | Grassland (winter only) |  |
| Whooper Swan | Cygnus cygnus | - | I | II | P | II | II | Grassland (winter only) |  |
| Greenland White-fronted Goose | Anser albifrons flavirostris | - | I | II | P | III | II | Grassland, bog (winter only) |  |
| Barnacle Goose | Branta leucopsis | - | I | II | P | II | II | Grassland, islands (winter only) | Mainly west coast |
| Light-bellied Brent Goose | Branta bernicla hrota | - | - | II | P | III | II | Estuary (winter only) | Coastal only |
| Hen Harrier | Circus cyaneus | - | I | E | P | III | II | Moorland, forestry, upland | Mainly south |
| Merlin | Falco columbarius | - | I | R | P | II | II | Moorland, forestry, upland |  |
| Peregrine | Falco peregrinus | - | I | II | P | II | II | Rock cliffs, quarries | Widespread |
| Grey Partridge | Perdix perdix | - | - | E | P | III | - | Farmland, bog | Localised |
| Corncrake | Crex crex | - | I | E | P | II | - | Farmland | Localised |
| Golden Plover | Pluvialis apricaria | - | I | V | P | III | II | Upland, bog | West and NW |
| Dunlin | Calidris alpina | - | - | V | P | III | II | Upland, bog, coastal |  |
| Black-tailed Godwit | Limosa limosa | - | - | R | P | III | II | Lowland wet grassland | Shannon callows |
| Greenshank | Tringa nebularia | - | - | R | P | III | II | Blanket bog | West and NW |
| Red-necked Phalarope | Phalaropus lobatus | - | I | E | P | II | II | Coastal marsh | NW Mayo only |
| Sandwich Tern | Sterna sandvicensis | - | I | - | P | # | # | Coastal islands | Coastal only |
| Roseate Tern | Sterna dougallii | - | I | E | P | II | II | Coastal islands | Coastal only |
| Common Tern | Sterna hirundo | - | I | - | P | # | # | Coastal and lake islands |  |
| Arctic Tern | Sterna paradisaea | - | I | - | P | # | # | Coastal islands | Coastal only |
| Little Tern | Sterna albifrons | - | I | V | P | II | - | Shingle beaches | Coastal only |
| Barn Owl | Tyto alba | - | - | I | P | II | - | Buildings | Widespread |
| Short-eared Owl | Asio flammeus | - | I | R | P | II | - | Coastal (winter only) |  |
| Nightjar | Caprimulgus europaeus | - | I | E | P | II | - | Upland, diverse | Localised |
| Ring Ouzel | Turdus torquatus | - | - | R | P | III | - | Upland | Localised |
| Wood Warbler | Phylloscopus sibilatrix | - | - | R | P | II | - | Woodland | Localised |
| Bearded Tit | Panurus biarmicus | - | - | R | P | II | - | Reedswamp | Localised |
| Tree Sparrow | Passer montanus | - | - | I | P | III | - | Buildings | Localised |
| Twite | Carduelis flavirostris | - | - | I | P | III | - | Upland and coastal | Mainly west |
| Corn Bunting | Miliaria calandra | - | - | E | P | III | - | Coastal scrub | West coast |
| Kingfisher | Alcedo atthis | - | I | - | P | # | # | Lowland river and lake | Widespread |
| Chough | Pyrrhocorax pyrrhocorax | - | I | II | P | II | - | Coastal cliffs and caves | Mainly W and S coasts |
| FISH |  |  |  |  |  |  |  |  |  |
| River Lamprey | Lampetra fluviatilis | II, V | - | I | - | III | - | Shallow inshore waters & accessible rivers | Unknown: north, south and south-east |
| Brook Lamprey | Lampetra planeri | II | - | I | - | III | - | Sandy, gravelly rivers and streams (limest.) | Unknown: north, n-west, south and s-east |
| Sea Lamprey | Petromyzon marinus | II | - | I | - | III | - | Deep offshore waters, shallow estuar. & riv. | Unknown: north, south and south-east |
| Sturgeon | Acipenser sturio | II, IV | - | - | - | - | # | no information |  |
| Atlantic Salmon (freshwater only) | Salmo salar | II, V | - | II | - | III | - | Rivers | Widespread and abundant |
| Allis Shad | Alosa alosa | II, V | - | E | - | III | - | Shallow coastal waters, estuaries, rivers | Unknown: north-west and south-east |
| Twaite Shad | Alosa fallax fallax | II, V | - | V | - | III | - | Sea, lower reaches of slow-flowing rivers | Unknown: south east coast |
| Killarney Shad | Alosa fallax killarnensis | II, V | - | E | - | III | - | Lakes | Kerry (Killarney lakes) |
| Arctic Charr | Salvelinus alpinus | - | - | V | - | - | - | Cool, stony, oligotrophic freshwater lakes | Mainly western lakes, some central and eastern |
| Pollan | Coregonus autumnalis pollan | V | - | E | - | III | - | Freshwater lakes | L. Neagh and Erne, possibly L. Ree and Derg |
| Smelt | Osmerus eperlanus | - | - | V | - | - | - | Estuaries and lower reaches of large rivers |  |
| REPTILES |  |  |  |  |  |  |  |  |  |
| Common Lizard | Lacerta vivipara | - | - | - | P | - | - | no information |  |
| AMPHIBIANS |  |  |  |  |  |  |  |  |  |
| Natterjack Toad | Bufo calamita | IV | - | E | P | II | - | Coastal sand dunes | Kerry |
| Common Frog | Rana temporaria | V | - | II | P | III | - | Wetlands | Widespread and common throughout Ireland |
| Smooth newt | Triturus vulgaris | - | - | - | P | - | - | no information |  |
| CRUSTACEANS |  |  |  |  |  |  |  |  |  |
| White-clawed Crayfish | Austropotamobius pallipes | II, V | - | - | P | # | # |  |  |
| INSECTS |  |  |  |  |  |  |  |  |  |
| Marsh Fritillary | Euphydryas aurinia | II | - | - | - | # | # |  |  |
| MOLLUSCS |  |  |  |  |  |  |  |  |  |
| Kerry Slug | Geomalacus maculosus | II, IV | - | - | P | # | # |  |  |
| Semi-aquatic snail | Vertigo angustior | II | - | - | - | # | # |  |  |
| Semi-aquatic snail | Vertigo geyeri | II | - | - | - | # | # |  |  |
| Semi-aquatic snail | Vertigo moulinsiana | II | - | - | - | # | # |  |  |
| Freshwater Pearl-mussel | Margaritifera margaritifera | II, V | - | - | P | # | # |  |  |
|  |  | EU HD | FPO 1999 | FPO 1980 | RDB |  |  |  |  |
| VASCULAR PLANTS |  |  |  |  |  |  |  |  |  |
| Basil-thyme | Acinos arvensis | - | + | + | V |  |  | Quaries, sandy gravelly places | Centre and S.E. rare |
| Moschatel | Adoxa moschatellina | - | - | - | V |  |  | Scrubby woodland | Belfast, Dublin |
| Pyramidal Bugle | Ajuga pyramidalis | - | - | - | R |  |  | Rocky ground | Galway Bay, Rathlin Is. V.rare |
| Alpine Lady's-mantle | Alchemilla alpina | - | - | - | R |  |  | Mountain cliffs | Kerry, Wicklow v. rare |
| Chives | Allium schoenoprasum | - | + | + | R |  |  | Woods, sandy ground | Cork, Kerry, Derry rare |
| Orange Foxtail | Alopecurus aequalis | - | + | - | ? |  |  | Muddy areas | South coast |
| Irish Sandwort | Arenaria ciliata | - | + | + | R |  |  | Steep grassy rocky slopes | Ben bulben, Co. Sligo |
| Arctic Sandwort | Arenaria norvegica | - | - | - | IN |  |  | Limestone rocks | N.W. Clare |
| Perennial Glasswort | Arthrocnemum perenne | - | + | + | V |  |  | Tidal mud | S. coast Wexford |
| Wild Asparagus | Asparagus officinalis | - | + | + | R |  |  | Sandhills | S. E. coast v. rare |
| Lanceolate Spleenwort | Asplenium obovatum (A. billotii) | - | + | + | V |  |  | Banks and walls | S & E coasts v. rare |
| Forked Spleenwort | Asplenium septentrionale | - | + | + | R |  |  | Volcanic rocks | Roundstone, Galway v. rare |
| Purple Milk Vetch | Astragalus danicus | - | + | + | R |  |  | Rocky sandy ground | Aran Islands |
| Smooth Brome | Bromus racemosus | - | - | - | R |  |  | Meadows, roadsides | Galway, + 8 counties |
| Bushgrass | Calamagrostis epigejos | - | + | + | R |  |  | Damp rocky places | W. & N. v. rare |
| Narrow Small-reed | Calamagrostis stricta | - | - | - | V |  |  | Lake shore | L. Neagh rare |
| Starwort | Callitriche truncata | - | + | - | R |  |  | lake, stream | Co. Wexford |
| Bats-in-the-belfry | Campanula trachelium | - | - | + | V |  |  | Wooded river banks | S.E. Nore valley, rare |
| Large Bitter-cress | Cardamine amara | - | - | - | R |  |  | Meadows riversides, alder woods | Ulster rare |
| Narrow-leaved Bitter-cress | Cardamine impatiens | - | + | + | R |  |  | Esker woodland | Westmeath v. rare |
| Northern Rock-cress | Cardaminopsis petraea | - | + | + | R |  |  | Mountain cliffs | Tipperary, Leitrim |
| Musk Thistle | Carduus nutans | - | - | - | IN |  |  | Pastures, heaths, roadsides | Dublin, Meath, Down |
| Starved Wood Sedge | Carex depauperata | - | + | + | R |  |  | Dry woods on limestone | Cork |
| Divided Sedge | Carex divisa | - | + | - | - |  |  | Estuarine marshes | River Barrow estuary |
| Tall Bog Sedge | Carex magellanica | - | - | - | R |  |  | Mountain bogs | Ulster rare |
| Fen-flowered Sedge | Carex pauciflora | - | - | - | V |  |  | Wet bogs | Antrim, Down |
| Cornflower | Centaurea cyanus | - | - | - | Ex* |  |  | Roadsides | East, v. rare |
| Seaside Centuary | Centaurium littorale | - | - | - | V |  |  | Dunes and sandy habitats | Known only in Co. Derry |
| Lesser Centaury | Centaurium pulchellum | - | + | + | V |  |  | Sandhills and sandy habitats | Waterford + north Dublin |
| Narrow-leaved Helleborine | Cephalanthera longifolia | - | + | - | V |  |  | Damp woodland and scrub | West Meath + western counties |
| Melancholy Thistle | Cirsium helenioides | - |  | - | R |  |  | Wet grassland | Fermanagh + Lough Gill |
| Autumn Crocus | Colchicum autumnale | - | + | + | E |  |  | Damp meadows and riverbanks | Nore Valley, v. rare |
| Parsley Fern | Cryptogramma crispa | - | + | + | R |  |  | Screes and rocky montane habitats | Galway, Down, Antrim, v. rare |
| Bog Hair Grass | Deschampsia setacea | - | + | + | RV |  |  | Wet bogs and lakesides | West Galway, v. rare |
| Hoary Whitlowgrass | Draba incana | - | - | - | R |  |  | Screes,cliffs and sandhills | Mainly in north-west |
| Eight-stamened Waterwort | Elatine hydropiper | - | - | - | R |  |  | Shallow lakes and rivers | Confined to north-east |
| Dwarf Spike-rush | Eleocharis parvula | - | - | - | V |  |  | Estuarine mud shores | Kerry, Wicklow, v. rare |
| Chickweed Willow-herb | Epilobium alsinifolium | - | + | + | R |  |  | Mountain streams + spring margins | Galway, Down, Antrim, rare |
| Green-flowered Helleborine | Epipactis phyllanthes | - | - | - | V |  |  | Sand dunes and open woodland | South-east + north,v. rare |
| Shady Horsetail | Equisetum pratense | - | - | - | R |  |  | Montane stream banks, glens + moors | Donegal |
| Moore's Horsetail | Equisetum X moorei | - | + | - | - |  |  | Sandhills + clay banks nr. Sea | East coast |
| Dorset Heath | Erica ciliaris | - | - | + | V |  |  | Bog margins | Roundstone bog is only site. |
| Mackay's Heath | Erica mackaiana | - | - | - | R |  |  | Blanket bog | Donegal,Galway |
| Cornish Heath | Erica vagans | - | - | - | V |  |  | Moorland | Fermanagh,Down + Antrim |
| Blue Fleabane | Erigeron acer | - | - | - | V |  |  | Eskers + dry calcareous grassland | Central + S.Eastern Ireland |
| Slender Cotton Grass | Eriophorum gracile | - | + | + | RV |  |  | Wet acid bogs + lake margins | West Galway, v. rare |
| Dropwort | Filipendula vulgaris | - |  | - | R |  |  | Rocky, limestone heaths | S Galway, N Clare, v. rare |
| Alder Buckthorn | Frangula alnus | - |  | - | R |  |  | Rocky and boggy habitats | Mainly in west |
| Narrow-leaved Hemp Nettle | Galeopsis angustifolia | - | + | + | V |  |  | Eskers, arable fields,waste places | Mainly in South-east |
| Wood Cranesbill | Geranium sylvaticum | - | - | - | V |  |  | Scrub, meadows,damp woods, rock-ledges | Antrim |
| Little Robin | Geranium purpureum | - | - | - | V |  |  | Old walls + roadsides | South coast, v.rare |
| Round-leaved Cranesbill | Geranium rotundifolium | - | - | - | V |  |  | Roadsides, walls + hedges | Cork + Wexford |
| Opposite-leaved Pondweed | Groenlandia densa | - | NOT Annex II spp as stated here before | + | V |  |  | Slow-flowing + standing water, estuarine mud | Limerick,Laois,Dublin,Antrim |
| Oak Fern | Gymnocarpium dryopteris | - | - | - | V |  |  | Shady, rocky montane habitats | Clare, Wicklow, Sligo, Leitrim, Antrim |
| Limestone Fern | Gymnocarpium robertianum | - | + | + | V |  |  | Limestone rocks and screes | Mayo, v. rare |
| Bog Orchid | Hammarbya paludosa | - | + | + | RV |  |  | Wet acid bogs | Single sites throughout Ireland, v. rare |
| Hoary Rockrose | Helianthemum canum | - | - | - | R |  |  | Limestone rocks and pasture | Clare and Galway, v. local |
| Common Rockrose | Helianthemum nummularium | - | + | + | R |  |  | Grassland overlying limestone | 1 site in Donegal |
| Holy-grass | Hierochloe odorata | - | - | - | R |  |  | Wet meadows and marshes | 1 site Lough Neagh, Antrim |
| Meadow Barley | Hordeum secalinum | - | + | + | V |  |  | Damp coastal and inland grasslands | East and south-east, rare |
| Wood barley | Hordelymus europaeus | - | - | - | IN |  |  | Shady river glen | Apparently extinct |
| Water-violet | Hottonia palustris | - | - | - | R |  |  | Ponds, ditches and marshes | 2 sites in Down (intr. Tipperary and Meath) |
| Irish Hydrilla | Hydrilla verticillata | - | + | + | V |  |  | Lakes | 1 lake in Galway |
| Hen-bane | Hyoscyamus niger | - | - | - | R |  |  | Sandhills, sandy open areas and wasteground | Mainly east coast, v. rare |
| Canadian St John's Wort | Hypericum canadense | - | + | + | R |  |  | Wet, boggy lake margins and heaths | 2 sites in Mayo and Cork |
| Hairy St John's Wort | Hypericum hirsutum | - | + | + | V |  |  | River banks and shady places | East, v. rare |
| Smooth Cat's-ear | Hypochoeris glabra | - | - | - | IN |  |  | Sand dunes | 4 sites in Derry and 1 in Antrim |
| Willow-leaved Inula | Inula salicina | - | + | + | V |  |  | Stony limestone shores | Lough Derg, Clare |
| Round-fruited Rush | Juncus compressus | - | - | - | R |  |  | Alluvial grassland | Meath, Roscommon, Longford, increasing |
| Fluellen | Kickxia elatine | - | - | + | V |  |  | Arable fields of root crops near coast | South and west, rare |
| Yellow Archangel | Lamiastrum galeobdolon | - | - | - | R |  |  | Woods and hedges | Mainly south-east, locally abundant |
| Sea Pea | Lathyrus japonicus | - | + | + | IN |  |  | Mainly maritime sands and shingles |  |
| Scot's Lovage | Ligusticum scoticum | - | - | - | R |  |  | Maritime cliffs and rocky shores | North-west, rare |
| Mudwort | Limosella aquatica | - | + | - | R |  |  | River, lake, reservoir and turlough margins | West, v. local |
| Slender Cudweed | Logfia minima | - | + | + | R |  |  | Sandy and gravelly places | Mainly south and east, rare |
| Darnel | Lolium temulentum | - | - | - | Ex* |  |  | Cultivated fields and waste ground | Apparently extinct |
| Hairy Birdsfoot Trefoil | Lotus subbiflorus | - | + | + | R |  |  | Dry and rocky grasslands near coast | West Cork and Wexford, v. rare |
| Marsh Clubmoss | Lycopodiella/Lepidotis inundata | - | + | - | R |  |  | Lake margins and wet bogs | Mainly west and north, v. rare |
| Small Cow-wheat | Melampyrum sylvaticum | - | - | - | V |  |  | Upland woodlands | North-east, v. rare |
| Penny Royal | Mentha pulegium | - | + | + | V |  |  | Damp sandy places | 3 sites: 1 in Cork, Mayo and Antrim/Derry border |
| Oyster Plant | Mertensia maritima | - | + | + | R |  |  | Gravelly seashores and shingle beaches | North and north-east coasts, v. rare |
| Recurved Sandwort | Minuartia recurva | - | + | + | R |  |  | Bare siliceous soils | 2 sites in south-west |
| Lesser Snapdragon | Misopates orontium | - | + | + | V |  |  | Cultivated fields | 2 sites in south and south-east |
| Yellow Bird's-nest | Monotropa hypopitys | - | - | - | R |  |  | Mainly beech and pine woods | 2 sites in Galway and 1 in Wexford |
| Slender Naiad | Najas flexilis | II, IV | + | + | RV |  |  | Lacustrine lakes and deep water | West, rare but J207increasing |
| Corky-fruited Water Dropwort | Oenanthe pimpinelloides | - |  | + | R |  |  | Damp grassland | Clare, rare |
| Wood Cudweed | Omalotheca sylvatica | - | + | + | R |  |  | Upland pastures and damp sandy places | Mainly north, rare |
| Green-winged Orchid | Orchis morio | - | - | + | V |  |  | Meadows, pastures and sandhills | Mainly in centre and east, rare (delisting?) |
| Birdsfoot | Ornithopus perpusillus | - | - | + | R |  |  | Dry sandy and gravelly places | South and east coasts, v. rare |
| Greater Broomrape | Orobanche rapum-genistae | - | - | - | R |  |  | Parasite of Ulex and Cytisus | South and east coasts, v. rare |
| Serrated Wintergreen | Orthilia secunda | - | - | - | E |  |  | Bogs and wet mountain ledges | 1 site in Fermanagh |
| Cottonweed | Otanthus maritimus | - | + | + | E |  |  | Sandy seashores and stable shingle | 2 sites in Wexford |
| Round Prickly-headed Poppy | Papaver hybridum | - | + | + | E |  |  | Sandy and gravelly places | 1 site in Dublin |
| Pillwort | Pilularia globulifera | - | + | + | RV |  |  | Margins of acid lakes and rivers | Mainly west coast, v. rare |
| Alpine Meadow-grass | Poa alpina | - | - | - | R |  |  | Mountain cliff edges | 3 sites in west (Kerry and Sligo), v. rare |
| Swamp Meadow-grass | Poa palustris | - | - | - | R |  |  | Damp places along rivers and canals | Mainly in north, rare |
| Sea Knotgrass | Polygonum maritimum | - | - | + | R |  |  | Sandy seashore | 1 site in Waterford |
| Alpine Bistort | Polygonum viviparum | - | + | + | IN |  |  | Mountain ridges and summits | Kerry and north-west, rare |
| Holly Fern | Polystichum lonchitis | - | - | - | R |  |  | Basic rock on mountain cliffs | West, v. rare |
| Shrubby Cinquefoil | Potentilla fruticosa | - | - | - | R |  |  | Rocky places subject to flooding | West, locally frequent |
| Small White Orchid | Pseudorchis albida | - | + | + | V |  |  | Upland pastures and heaths | North, v. rare |
| Tufted Salt-marsh Grass | Puccinellia fasciculata | - | + | + | R |  |  | Salt marshes, muddy inlets and estuaries | South and east coasts, v. rare |
| Intermediate Wintergreen | Pyrola media | - | - | - | R |  |  | Woods, shady glens, heaths and rocky places | North and west, rare |
| Round-leaved Wintergreen | Pyrola rotundifolia ssp. maritima | - | + | + | RV |  |  | Damp hollows in dunes | 1 site in Wexford |
| Three-lobed Water Crowfoot | Ranunculus tripartitus | - | - | + | R |  |  | Nutrient-poor, temporary pools and ditches | South-west, v. rare |
| River Water-Crowfoot | Ranunculus fluitans | - | - | - | R |  |  | Rivers | 1 site in Antrim |
| Northern Yellowcress | Rorippa islandica | - | - | - | R |  |  | Turloughs | Clare and Galway, v. rare |
| Cloudberry | Rubus chamaemorus | - | - | - | V |  |  | Montane moorland | 1 site in Tyrone |
| Golden Dock | Rumex maritimus | - | - | - | R |  |  | Pools, lake margins and wet hollows | 4 sites: 1 in Cork, Wexford, Limerick and Sligo |
| Tea-leaved Willow | Salix phylicifolia | - | - | - | R |  |  | Wet mountain cliffs and streamsides | West and north, v. rare |
| Wild Sage | Salvia verbenaca | - | - | - | R |  |  | Dry sandy banks and waste places near coast | Southern half, from Dublin to Galway, rare |
| Great Burnet | Sanguisorba officinalis | - | + | + | V |  |  | Lake shores and dry banks | Mayo, Antrim and Down, v. rare |
| Alpine Saw-wort | Saussurea alpina | - | - | - | R |  |  | Mountain cliffs and ledges over 300m | Mainly in west from Donegal to Kerry, v. rare |
| Yellow Saxifrage | Saxifraga aizoides | - | - | - | R |  |  | Damp rocky places | North-west, rare |
| Meadow Saxifrage | Saxifraga granulata | - | + | + | E |  |  | Sandhills and pastures near coast | East, v. rare (note: introduced in places) |
| Hart's Saxifrage | Saxifraga hartii | - | + | + | R |  |  | Sea cliffs | 1 site in Donegal |
| Yellow Marsh Saxifrage | Saxifraga hirculus | II, IV | + | + | E |  |  | Wet bogs | 2 sites: 1 in Mayo and Antrim, v.rare |
| Alpine Saxifrage | Saxifraga nivalis | - | + | + | R |  |  | Mountaincliffs and damp rocks | 1 site in Sligo |
| Purple Saxifrage | Saxifraga oppositifolia | - |  | - | R |  |  | Mountain rocks | North and west, rare |
| Triangular Club Rush | Scirpus triqueter | - | + | + | V |  |  | Muds bordering tidal rivers | Limerick, locally abundant |
| Annual Knawel | Scleranthus annuus | - | + | - | - |  |  | Dry sandy waste places and roadsides | Mainly north-east, rare |
| Green Figwort | Scrophularia umbrosa | - | - | - | V |  |  | River banks and lake shores | 1 site in Derry (possibly Dublin/Kildare) |
| Cornish Moneywort | Sibthorpia europaea | - | - | - | R |  |  | Streamsides and damp grassland | Kerry, occasional |
| Moss Campion | Silene acaulis | - | - | - | R |  |  | Mountain slopes and cliffs | North-west, rare |
| Kerry Lily | Simethis planifolia | - | + | + | V |  |  | Dry heathy ground near the sea | 1 site in Kerry |
| Drooping Lady's Tresses | Spiranthes romanzofiana | - | + | + | R |  |  | Damp meadows, lake shores, boggy ground | South-west, west, north-east, rare and v. local |
| Woundwort | Stachys officinalis | - | + | + | V |  |  | Open woods, hedges and grasslands | Scattered throughout, v. rare and local |
| Gotlandian Dandelion | Taraxacum gotlandicum | - | - | - | R |  |  | Sandhills | 1 site in Clare |
| Shepherd's Cress | Teesdalia nudicaulis | - | - | - | R |  |  | Sandy lake shores | Scattered in north and south-west, v. rare |
| Killarney Fern | Trichomanes speciosum | II, IV | + | + | RV |  |  | Damp, dark, sheltered places | Mainly in west, isolated eastern sites, rare |
| Clustered Clover | Trifolium glomeratum | - | + | + | V |  |  | Sandy/gravelly ground near coast | East and south, v. rare |
| Subterranean Clover | Trifolium subterraneum | - | + | + | V |  |  | Sandy/gravelly banks | 2 sites in Wicklow |
| Globe Flower | Trollius europaeus | - | + | + | V |  |  | Mainly lake shores and river banks | 4 sites: 2 in Donegal and 2 in Fermanagh |
| Spotted Rock-rose | Tuberaria guttata | - | - | - | R |  |  | Rocky, maritime heathland | West and south-west, v. local |
| Bitter Vetch | Vicia orobus | - | + | + | V |  |  | Thickets and rocky ground | West, centre and north-east, rare |
| Hairy Violet | Viola hirta | - | + | + | V |  |  | Dry limestone banks, rocky ground, and scrub | South half of country, rare |
| Pale Heath Violet | Viola lactea | - | + | + | V |  |  | Heathy ground | South half, rare |
| Fen Violet | Viola persicifolia | - | - | - | R |  |  | Damp grassland subject to flooding | Clare & Galway (poss. Fermanagh), v. local |
| Spring Vetch | Vicia lathyroides | - | - | - | R |  |  | Sandy ground near the sea | East and north, rare |
| Paradoxical Rock Sea Lavender | Limonium paradoxum | - | - | - | - |  |  | Maritime cliffs | 1 site in Donegal |
| MOSSES |  |  |  |  |  |  |  |  |  |
| A moss | Bryum calophyllum | - | + | - | - |  |  | Damp calcareous dune slacks | W. Mayo |
| A moss | Bryum marratii | - | + | - | - |  |  | Damp dune slacks | W. Mayo, W. Donegal, Londonderry |
| A moss | Catoscopium nigritum | - | + | - | - |  |  | calcicole, montane flushes, dune slacks | N. coast of Ireland |
| Shining sicklemoss | Drepanocladus vernicosus | II | + | - | - |  |  | ?? Bogs, fens flushes |  |
| A moss | Leptobarbula berica | - | + | - | - |  |  |  |  |
| A moss | Orthrotrichum pallens | - | + | - | - |  |  | Small tufts on banks |  |
| A moss | Orthrotrichum sprucei | - | + | - | - |  |  |  |  |
| A moss | Orthrotrichum stramineum | - | + | - | - |  |  | Tufts on bark |  |
| A moss | Paludella squarrosa | - | + | - | - |  |  | Tufts on fens |  |
| A moss | Pottia wilsonii | - | + | - | - |  |  | Disturbed ground on coastal habitats |  |
| A moss | Tetraplodon angustatus | - | + | - | - |  |  | Dung in moist montane habitats |  |
| A moss | Tortella inclinata | - | + | - | - |  |  | Basic soil in turf, on sandunes | W. Mayo, Down |
| A moss | Weissia longifolia | - | + | - | - |  |  | Non-calcareous soild in arable fields& limestone grassland |  |
| A moss | Weissia rostellata | - | + | - | - |  |  | Damp clay in arable fields, edge of ponds |  |
| LIVERWORTS |  |  |  |  |  |  |  |  |  |
| A liverwort | Leiocolea gillmanii | - | + | - | - |  |  |  |  |
| Fen flapwort | Leiocolea rutheana | - | + | - | - |  |  |  |  |
| Petalwort | Petalophyllum ralfsii | II | + | - | - |  |  |  |  |
| A liverwort | Plagiochila atlantica | - | + | - | - |  |  |  |  |
| LICHENS |  |  |  |  |  |  |  |  |  |
| A lichen | Fulgensia fulgens | - | + | - | - |  |  |  |  |
| STONEWORTS |  |  |  |  |  |  |  |  |  |
| A stonewort | Chara canescens | - | - | - | V |  |  | Brackish waters with highpH | Scattered around west and south coasts |
| A stonewort | Chara connivens | - | - | - | I |  |  | Alkaline lakes, ponds and ditches | Clare, Galway, Wexford (all before 1970) |
| A stonewort | Chara denudata | - | - | - | R |  |  | Calcareous lakes | 4 sites in Westmeath |
| A stonewort | Chara muscosa | - | - | - | Ex |  |  | Sand at shallow margins of lakes | Apparently extinct (possibly Donegal) |
| A stonewort | Chara tomentosa | - | - | - | R |  |  | Medium to large calcareous lakes | Scattered in midlands and west |
| Foxtail Stonewort | Lamprothamnium papulosum | - | + | - | V |  |  | Natural and artificial brackish lagoons | 3 sites in Wexford and Clare |
| Slender Stonewort | Nitella gracilis | - | + | - | V |  |  | Mainly shallow ditches, flushes and pools | 1 possibly 2 sites in Wicklow |
| A stonewort | Nitella mucronata | - | - | - | R |  |  | Mesotrophic-eutrophic water | 2 sites in Fermanagh and 1 in Dublin |
| A stonewort | Nitella spanioclema | - | - | - | I |  |  | Clear mesotrophic water | Apparently extinct (possibly Donegal) |
| A stonewort | Nitella tenuissima | - | - | - | V |  |  | Peaty pools and ditches of calcareous fens | Clare, possibly Galway and Westmeath |
| A stonewort | Tolypella intricata | - | - | - | V |  |  | Pools, canals, ditches of alkaline waters | Dublin only |
| A stonewort | Tolypella prolifera | - | - | - | Ex |  |  | Ditches, rivers, canals of small alkaline waters | Apparently extinct (possibly Dublin) |

## Appendix 6:  Site synopsis for designated sites of conservation interest in Co. Wicklow

SITE SYNOPSIS

SITE NAME:  CARRIGGOWER BOG

SITE CODE:  000716

Carriggower Bog is situated on Calary plateau at the eastern edge of the Wicklow Mountains.  The site is an area of wet bog and poor fen, flanked by the Vartry River on the south-western side.

This site is a candidate SAC selected for transition mire, a habitat listed on Annex I of the EU Habitats Directive.

The bog was exploited for peat extraction until 100 years ago and the peat cuttings are now flooded.  The remaining bog vegetation is characterised by bog moss hummocks.  Several species of Sphagnum are found including Sphagnum recurvum.  On top of the hummocks Heather (Calluna vulgaris) and Cross-leaved Heath (Erica tetralix) are dominant.  Between the hummocks, in the wet areas, Bog Asphodel (Narthecium ossifragum), Bogbean (Menyanthes trifoliata) and Common Cottongrass (Eriophorum angustifolium) are common, while Purple Moor-grass (Molinia caerulea) is locally abundant.

Birch (Betula pubescens) and Willow (Salix spp.) dominate an area of scrub at the centre of the site.

Very wet areas of transition mire occur on the south-western side of the site.  The whole area is quaking and is characterised by a mosaic of sedges, grasses and rushes.  Bottle Sedge (Carex rostrata) is the most abundant sedge, but others include White Sedge (Carex curta), Star Sedge (Carex echinata) and Carnation Sedge (Carex panicea).  Purple Moor-grass and Sharp-flowered Rush (Juncus acutiflorus) are also present.

Towards the northern side of the site there are rock outcrops of quartzite which form low knolls.  This area is partly covered by heath vegetation, dominated by Gorse (Ulex europaeus), Bell Heather (Erica cinerea) and Heather, all of which are growing over a grassy sward grazed by sheep.  The main grasses are Sweet Vernal-grass (Anthoxanthum odoratum), Yorkshire-fog (Holcus lanatus), Creeping Soft-grass (Holcus mollis) and Bent Grass (Agrostis spp.)

The mosaic of transition bog habitats supports a wide range of poor fen/bogland invertebrates, including a suite of wetland species of international importance (most of the Order Diptera).

Carriggower Bog is a notable site for wintering Snipe (117 – mean winter count 1996/97 to 2000/01) and Jack Snipe (16 – mean winter count 1996/97 to 2000/01).  These numbers are probably of national importance and the site has consistently held the highest national count for Jack Snipe in recent years.  The very wet bog is prime habitat for both these species.

This site is of conservation importance because it shows a good transition between fen and bog vegetation (with the fen being rapidly colonised by characteristic bog species).  The area holds a rich and varied flora and it provides a habitat for some rare invertebrates; Carrigower Bog is the last remaining site in Wicklow from which some of these remaining invertebrates are recorded.

SITE SYNOPSIS

SITE NAME:  THE MURROUGH WETLANDS

SITE CODE:  002249

The Murrough is a coastal wetland complex which stretches for 15 km from Ballygannon to north of Wicklow town, and in parts, extends inland for up to 1 km.  A shingle ridge stretches the length of the site and carries the mainline Dublin-Wexford railway.

The site supports a number of habitats listed on Annex I of the EU Habitats Directive and a number of bird species listed on Annex I of the EU Birds Directive, as well as a wide range of important migratory birds.  There are also many rare plants in the site.

On the seaward side, driftline vegetation includes species such as Sea Rocket (Cacile maritima), Sea Sandwort (Honkenya peploides), Sea Holly (Eryngium maritimum) and Yellow-horned Poppy (Glaucium flavum).  The rare and legally protected Oyster Plant (Mertensia maritima) (Flora (Protection) Order, 1999) has been recorded on the gravelly shore in the past but is now considered to be extinct from this locality.

Low sand hills occur at Kilcoole, with Marram (Ammophila arenaria) and Lyme-grass (Leymus arenarius).  In other areas and further inland a rich grassy sward, which is most extensive in the south end of the site, has developed.  Typical species include Sweet Vernal-grass (Anthoxanthum odoratum), Crested Dog’s-tail (Cynosurus cristatus), Common Bird’s-foot-trefoil (Lotus corniculatus), Burnet Rose (Rosa pimpinellifolia) and Pyramidal Orchid (Anacamptis pyramidalis).  A community dominated by Silverweed (Potentilla anserina) and Strawberry Clover (Trifolium fragiferum) occurs in some of the wetter, grassy areas.  In some places, particularly at the south of the site, a Gorse (Ulex) heath has developed on the stony ridge. During site selection, both local and regional rationale will be considered.

Saltmarsh is present within the site in two distinct areas.  At the southern end of the site, Broad Lough, a brackish, partly tidal lake, has a well developed saltmarsh community which includes Sea Rush (Juncus gerardii), Common saltmarsh-grass (Puccinellia maritima), Sea Aster (Aster tripolium), Sea Purslane (Hamilione portulacoides) and Common Scurvy-grass (Cochlearia officinalis).  Common Reed (Phragmites australis) is abundant along the western shore, along with some Sea Club-rush (Scirpus maritimus)The regional considerations are used to identify criteria for the selection of sites throughout Northern Ireland.  These reflect the features used to select ASSIs i.e. habitat, extent/population size, diversity, naturalness, rarity and typicalness.

Saltmarsh is also present in the northern end of the site in the vicinity of the Breaches. Though this has been greatly affected by drainage in the late 1980s and early 1990s, localised Sea Couch (Elymus pycnanthus) still occurs.  The grassland which was improved as a result of the drainage is now influenced by seepage and flooding of saline waters.Local consideration is given to ensure the selection of sites is modified to take into account geographical distribution and local requirements.

An area of fen occurs at Five Mile Point.  Here Black Bog-rush (Schoenus nigricans) is dominant, with Marsh Pennywort (Hydrocotyle vulgaris), Purple Moor-grass (Molinia caerulea), heather (Calluna vulgaris), Cross-leaved heath (Erica tetralix), Devil’s-bit Scabious (Succisa pratensis) and a wide variety of orchids also present.  The rare, Narrow-leaved Marsh Orchid (Dactylorhiza traunsteineri) has also been recorded here.  Fen Sedge (Cladium mariscus) is present where the ground is wetter.  This in turn, merges into areas dominated by Common Reed.  Fen is found in mosaic with reed bed, and wet woodland in the townland of Blackditch.

A fine wet woodland occurs at Blackditch.  Birch (Betula pubescens) is the dominant species with some Alder (Alnus glutinosa), Willow (Salix spp.) and Ash (Fraxinus excelsior) also present.  The ground flora of this wooded area is often quite dense.  This wood also contains a rich invertebrate community with at least eight rare or notable species of fly (Diptera) occurring, including Syntormon setosus, a species unknown elsewhere in Britain or Ireland.

A wide range of freshwater and brackish marsh habitats occur within the site.  These vary from reed-marsh dominated by reeds and Rushes (Juncus spp.), to those of Sedges (Carex spp.) with other areas supporting a mixture of Sedges and Yellow Iris (Iris pseudacorus) also occurring.  A wide variety of grasses and herbs are also found.  These include Meadowsweet (Filipendula ulmaria), Silverweed and Common Spike-rush (Eleocharis palustris).  The scarce, Marsh Pea (Lathyrus palustris) occurs in one area.  The marshes merge into wet grassland in many areas.  Where grazing pressure is low, a herb-rich sward occurs with species such as Ragged Robin (Lychnis flos-cuculi), Cuckoo Flower (Cardamine pratensis), Meadowsweet and Spotted Orchid (Dactylorhiza maculata) occurring.  Sedges are abundant in the wetter areas.  Where drains have been cut, there are many other species such as Greater Spearwort (Ranunculus lingua), Bogbean (Menyanthes trifoliata) and the scarce Reed Sweet-grass (Glyceria maxima).

The Murrough is an important site for wintering waterfowl and breeding birds.  Annex I bird species present include Red-throated Diver, Little Egret, Bewick’s Swan, Whooper Swan, Greenland White-fronted Goose, Golden Plover, Kingfisher, Sandwich Tern and Little Tern.  Average peak winter counts from 1994/95 - 1997/98 showed the site to have an internationally important population of Brent Geese (1,318, which is much higher than it was in the early 90s), nationally important populations of Wigeon (1,518), Teal (772), Common Scoter (103) and Lapwing (3,140) and regionally or locally important populations of Whooper Swan (80), Little Grebe (22), Shelduck (95), Gadwall (9), Mallard (391), Shoveler (22), Golden Plover (615), Curlew (605) and Redshank (181).  Greylag Geese numbers were nationally important in the early 90s but these numbers have dropped off.  The average peak is now 213.

Little Tern breed on the shingle beach near The Breaches and this is the largest colony on the east coast (c.50 pairs in 1993, an average of 37 pairs over the ten year period 1988-1998 ).  Redshank, Oystercatcher, Ringed Plover and Water Rail also breed.  The reedbeds at Broad Lough provide habitat for Reed Warbler and the rare Bearded Tit has bred here.  Otter has been reported regularly from the Murrough.

Recent farming and drainage practices and afforestation have greatly reduced the area and quality of the wetlands habitats - the area between Kilcoole and Newcastle is particularly affected.  In 1997 there was some levelling of the sand hills below Killoughter station.  Pollution, reclamation and further drainage would adversely affect this site.

This site is of importance as it is the largest coastal wetland complex on the east coast of Ireland.  Although much affected by drainage, it still contains a wide range of coastal and freshwater habitats including five listed on Annex I of the EU Habitats Directive, some of which contain threatened plants.  Areas on the site contain a rich invertebrate fauna, including several rarities.  It is an important site for both wintering and breeding birds and supports a wide variety of species listed on Annex I of the EU Birds Directive.

## Appendix 7:  Site synopsis for designated sites of conservation interest in Co. Westmeath

SITE SYNOPSIS

SITE NAME:  BALLYNAFID LAKE AND FEN

SITE CODE:  000673

Ballynafid lake is a small midland lake, some 10 hectares in extent, located just north-east of Lough Owel.  The lake is fringed by a wide band of reedswamp dominated by common reed (Phragmites australis) and Common Club-Rush (Scirpus lacustris).  The reedswamp grades into freshwater marsh containing Bottle Sedge (Carex rostrata) and Greater Spearwort (Ranunculus lingua) with a band of calcareous fen dominated by Tawny Sedge (Carex hostiana) and Purple Moor Grass (Molinia caerulea) behind the freshwater marsh.  On the western side of the lake, reedswamp grades into woodland dominated by Birch (Betula pubescens) with some Willows (Salix spp.) and Pine (Pinus sp.)  Bramble (Rubus fruticosus agg.) dominates a rather species-poor ground flora.  The lake is a popular angling venue with a good population of coarse fish (Bream, Rudd etc.) and this is mainly due to its close proximity to the road.

Ballynafid lake is a very important site for rare invertebrates and insects some of which are internationally rare.  The lake also displays nice patterns of vegetation zonation which are rarely seen so well-developed in midland lakes.  The transition from marsh to calcareous fen, particularly along the south-eastern edge of the lake, is of great interest.  Plant species of restricted distribution occurring at the site include Fen bedstraw (Galium uliginosum) and Marsh Pea (Lathyrus palustris) the latter species being only recently removed from the list of protected plant species.  Although the lake is too small to have much ornithological interest it does support a large population of Snipe along with some Mallard and Mute Swan.

SITE SYNOPSIS

SITE NAME:  LOUGH DERRAVARAGH

SITE CODE:  000684

Lough Derravaragh is located approx. 12km north of Mullingar town.  The River Inny, which is a major tributary of the River Shannon, flows into and out of the lake at its north-west end.  At this end, the lake is wide and shallow and is backed by raised or cutaway bog.  In contrast, the lake's south-eastern arm is narrow, running between resistant limestone outcrops which slope steeply to the lakeshore.  The lake is shallow and its water is hard with an alkaline pH.

A notable feature of Lough Derravaragh is the range of aquatic lower plant (Charophyte) species which occur here.  Eight species have been recorded, several of which are restricted in their distribution in Ireland.  Around the lake margin, a range of habitats have been created as a result of drainage of the River Inny.  At the western end are extensive reedbeds and swamps dominated by Common Reed (Phragmites australis) with scattered stands of Downy Birch (Betula pubescens) and Willows (Salix spp.).  Elsewhere, there is freshwater marsh vegetation dominated by sedges (Carex spp.) and often tussock-forming grasses (e.g. Deschampsia caespitosa and Festuca sp.), with a range of flowering herbs including Nodding Bur-Marigold (Bidens cernua) and Trifid Bur-Marigold (Bidens tripartita).  The lakeshore is a mineral-rich substrate and thus several plant species of poor fen habitats are abundant, such as Black Bog-rush (Schoenus nigricans) and Long-stalked Yellow Sedge (Carex lepidocarpa).

Small areas of raised bog adjoin the lake at its western end.  Here, the wet surface mostly consists of an undulating carpet of Sphagnum moss, Ling Heather (Calluna vulgaris) and Hares-tail Cottongrass (Eriophorum vaginatum) are abundant.  Cutaway areas are abundantly colonised by Scot's Pine (Pinus sylvestris), Downy Birch (Betula pubescens) with occasional Sessile Oak (Quercus petraea).

Knockeyon and the other hills around the south-eastern end of the lake support deciduous woodland which comprises mostly native species.  Hazel (Corylus avellana), Rowan (Sorbus aucuparia), Ash (Fraxinus excelsior) and Sessile Oak (Quercus petraea) are abundant.  Exotic species occur occasionally, including Beech (Fagus sylvatica).

Chara denudata, an aquatic charophyte species, has been recorded in Lough Derravaragh.  This rare plant occurs in only five sites in Co. Westmeath, is restricted to highly calcareous lakes and is intolerant of pollution.  The

Lough Derravaragh holds nationally important numbers of Pochard (average peak 933), Tufted Duck (average peak 451), Coot (average peak 315) and Mute Swan (average peak 137) (counts from 1984/5 to 1986/7).

The following insects have been recorded in Lough Derravaragh (all three are in Order Trichoptera): Metatype fragilis and Limnephillis nigriceps, also Tinodes maculicornis which is of Europeanof biodiversity significance.

This site is a valuable habitat for Otter, a species listed in Annex II of EU Habitats Directive. SPP3: Site which supports species assemblages of biodiversity significance.

The lake is an important amenity for anglers, as it holds a population of Brown Trout (Salmo trutta).  Knockbody Wood is used for shooting Pheasant.  Local groups use the lake for canoeing and watersports.  The fish and wildfowl are threatened by pollution of the lake with organic farm material, and a degree of eutrophication is indicated by the presence of green algae along Lough Derravaragh's shoresSPP4: Site which is important for populations of species (e.g. birds).

This lake is significant as a habitat for wildfowl, fish and otters, as well as for lower plants and insects, some of which are of European significance.  Although bird numbers have declined significantly since the 1970's, Lough Derravaragh's remaining wildfowl populations are less erratic than on other lakes in the locality, so its importance is still considerable.

SITE SYNOPSIS

SITE NAME:  LOUGH OWEL

SITE CODE:  000688

Lough Owel is a large (1008ha) calcareous lake located north-west of Mullingar.  It is a relatively shallow lake with a rocky, marl-covered bottom.  The rocky nature of the shoreline has given rise to marginal vegetation which is patchy and sparse.  Apart from some reedswamp formed by Common Reed (Phragmites australis) and Common Clubrush (Scirpus lacustris), shoreline vegetation is dominated by occasional patches of Alders (Alnus glutinosa).  There are however areas of marsh and fen in the northern and sout-western corners of the lake.  These areas (Bunbrosna marsh and Tullaghan fen) were formerly separate A.S.I.s but have now been subsumed into the Lough Owel N.H.A..  Bunbrosna is an area of marsh and fen which is partially invaded by Birches (Betula pubescens) and Willows (Salix spp.).  The area contains some rare plant species, namely Marsh Pea (Lathyrus palustris), Marsh Fern (Thelypteris palustris) and the protected Round-leaved Wintergreen (Pyrola rotundifolia).  Tullaghan fen is an area of flooded cut-over bog which has developed a varied fen and marsh vegetation quite similar to Bunbrosna marsh.  Mud Sedge (Carex limosa), Greater Tussock Sedge (Carex appropinquata) and Marsh Fern (Thelypteris palustris) are to be found here.  In addition, three other rare plant species are found along the lake margins namely, White Sedge (Carex curta), Marsh Stitchwort (Stellaria palustris) and Frogbit (Hydrocharis morsus-ranae).

Although Lough Owel is not noted for it's wildfowl there are small populations of Mallard, Shoveller, Pochard and Tufted duck present.  Farmland adjacent to the lake are feeding grounds for internationally important numbers of Greenland White-fronted Goose.  Lough Owel is one of the most important fishing lakes in the midlands and is especially good for trout.  The possible pressence of Scharff's char here (Savelinus scharffi), a distinct race confined to Lough Owel and Lough Ennell, is also of immense interest.

With the exception of Lough Carra in County Mayo, Lough Owel is the best example of a large, spring-fed calcareous lake in the country.  The lake and fringing wetland areas support an outstanding array of rare plant species as well as supporting bird and fish populations of considerable interest.  Possible threats to the scientific interest of the lake include the increasing of levels of water supply to Mullingar, overfishing, eutrophication caused by local farming practices and pressure from amenity uses such as boating and fishing.

SITE SYNOPSIS

SITE NAME:  SCRAGH BOG

SITE CODE:  000692

Scragh Bog Natural Heritage Area lies some 10km northwest of Mullingar.

This area is a wet transition fen with a floating root mat developed in a small oval-shaped depression.  The fen is fed by weak surface springs and drains by an artificially defined outlet.

The vegetation is mainly calcareous fen which becomes open carr in the central area and transitions to ombrotrophic bog.

Most of the fen vegetation belongs to two broad categories.  The first is dominated by Schoenus nigricans, with Carex lepidocarpa, Dactylorhiza traunsteineri, Triglochin palustris, Parnassia palustris, Campylium stellatum, Scorpidium scorpioides and Drepanocladus revolvens.  The second category is quaking fen in which Carex lasiocarpa is dominant and is associated with Menyanthes trifoliata, Equisetum fluviatile, Carex lepidocarpa, Drepanocladus revolvens, Bryum pseudotriquetrum and Cinclidium stygium.  Eriophorum gracile, a protected species of cotton-grass which is also rare in Europe, occurs in this vegetation type.  A third category of fen vegetation is dominated by large sedges, such as Carex appropinquata.

The fen carr is dominated by Salix spp. and by Betula pubescens.  Pyrola rotundifolia ssp. rotundifolia, another Red Data Book species (but no longer protected), is found in this vegetation type.

The embryonic raised bog communities contains such species as Carex limosa, Carex lasiocarpa, Erica tetralix, Drosera rotundifolia, Vaccinium oxycoccos, Aulacomnium palustre, Sphagnum plumulosum and Sphagnum contortum.

The remaining terrestrial vegetation types (apart from some planted coniferous forestry at the southern end) are two grassland communities: the first is a tall meadow community in which Filipendula ulmaria is dominant; the second is characterised by Molinia caerulea and Succisa pratensis.

Two aquatic communities are also found:  one free-floating, in which Lemna minor is prominent; and a formation of Chara spp.

Other rare plants found at the site include the arctic-alpine moss, Homalothecium nitens.

The site also supports a uniquely complete fauna of transition mire invertebrates, including a number of species which are extremely rare in Northern Europe.  Among the aquatic/subaquatic insects, Chrysops sepulchralis, Coenagrion hinulatum and Tetanocera freyi provide examples of rare Northern European species.  Acrometopia wahlbergi and Platycheinis perpallidus are rare species more closely associated with mire vegetation.  The fen carr also has its own complement of associated invertebrates of scientific interest.

Most of the site is managed as a Nature Reserve.  The outflow stream is included in the NHA, since interference with this outflow could damage the site hydrology.   A small section at the bottom of a field to the south is also included in the NHA - this area supports a species-rich marsh/wet grassland vegetation.

## Appendix 8.  The Irish Geological Heritage Programme

The Irish Geological Heritage (IGH) Programme is a partnership between The Geological Survey of Ireland (GSI) and the National Parks and Wildlife Service of The Department of Environment, Heritage and Local Government.

It aims to identify, document, and protect the wealth of geological heritage in the Republic of Ireland and conserve it against ever increasing threats, and also to promote its value with the landowners and the public.

The GSI provide scientific appraisal and interpretative advice on geological and geomorphological sites.  They are responsible for the identification of important sites that are capable of being conserved as Natural Heritage Areas (NHA).

The National Parks and Wildlife Service of The Department of Environment, Heritage and Local Government have the responsibility of designation and management of sites with appropriate advice from GSI.

The programme started only in the last few years and has evolved with examination of the inventory and protection processes operating in other European countries (including the active participation with colleagues in the European Association for the Conservation of the Geological Heritage, ProGEO).

The geological heritage of Ireland is considered and evaluated within an overall framework of sixteen themes:

IGH 1 - Karst (completed)
IGH 2 - Precambrian to Devonian Palaeontology (completed)
[including the Valentia tetrapod trackway site]
IGH 3 - Carboniferous to Pliocene Palaeontology (in progress)
IGH 4 - Cambrian-Silurian
IGH 5 - Precambrian
IGH 6 - Mineralogy
IGH 7 - Quaternary
IGH 8 - Lower Carboniferous 
IGH 9 - Upper Carboniferous and Permian
IGH 10 - Devonian
IGH 11 - Igneous Intrusions
IGH 12 - Mesozoic and Cenozoic
IGH 13 - Coastal Geomorphology
IGH 14 - Fluvial and Lacustrine Geomorphology
IGH 15 - Economic Geology
IGH 16 - Hydrogeology
[including warm springs, see: http://www.ecoserve.ie/projects/springs/index.html]

Each theme considers specific aspects, such as stratigraphy, sedimentology, structural geology, volcanic rocks, etc. Each theme addresses all aspects of the particular geology involved, but some sites may be considered within two or more themes.

The IGH Programme is now completing a process of expert panels for each theme in order to derive an indicative list of sites which are worthy of more detailed assessment within the relevant themes. This is a response to the present urgent demands for information for development projects, Environmental Impact Statements and planning.

Of the majority of geological sites not eventually selected for NHA designation, some are being promoted as County Geological Sites, which have no statutory protection, but may be included within County Development Plans. Many counties have now adopted County Geological Sites into their development plans, and are promoting their interest through Heritage Plans

Council Directive 92/43/EEC on the Conservation of Natural Habitats and of Wild Fauna and Flora. The Habitats Directive was transposed into Irish law under the European Union (Natural Habitats) Regulations, 1997 (S.I. 94 of 1997).

Council Directive 79/409/EEC on the Conservation of Wild Birds

Council Directive 2000/60/EC Establishing a Framework for Community Action in the Field of Water Poilcy